KWAME NKRUMAH UNIVERSITY OF SCIENCE AND

TECHNOLOGY

COLLEGE OF ENGINEERING

FACULTY OF MECHANICAL AND AGRICULTURAL ENGINEERING

DEPARTMENT OF AGRICULTURAL ENGINEERING

MECHANICAL ENGINEERING LAB II

TITLE : BELT FRICTION

GROUP 18

NAME                                                      INDEX NUMBER

PARKER NATHANIEL                                                     4856410

OWUSU-ANSAH ALBERT	                                        4856310

BOFFAH KOFI OBENG                                             4852310

OTOO NENYI OBED                                                  4855710

SUMMARY

The experiment was carried out to confirm the laws of belt friction and to determine the co-efficient of friction of the belt and pulley used. This was done by using the rotating cylinder apparatus, a peg, a piece of belt connected to a load hanger and a spring balance at different ends of the belt and weight(load). The peg is inserted in the first hole, and the spring balance with the belt and load hanger hanged on the peg, the belt is allowed to rest on the pulley in one of the four grooves. A thread is used to measure the circumference of the grooves of the pulley and the length that the belt covers is also measured, this was used to calculate the angle of lap θ. A load of 2lbf to 10lbf in an increment of 2lbf at a time was hanged on the load hanger and a reading T1 was taking from the spring balance in each case. A direction of rotation was selected and another reading T2 taking in each case this was repeated for three more different grooves. The whole process was repeated for two more holes at different angles. The readings T1, T2 and θ was used to calculate the effective co-efficient of friction µ.

INDIVIDUAL CONTRIBUTION

During the experiment PARKER NATHANIEL and OTOO OBED NENYI organized the report and recorded the data obtained in the experiment, OWUSU-ANSAH ALBERT also put the data obtained in a tabular form and BOFFAH OBENG KOFI was also rotating the cylindrical wheel throughout the experiment.

OBJECTIVE

- The experiment is intended to enable us to study the relationship between belt tensions, angle of lap and co-efficient of friction for flat and v belts.

- It further helps determine the effect of the angle of lap and belt tension on the power that can be transmitted for belt drive mechanism.

- The experiment lastly is to compare the power transmission compatibility of flat and v belts.

INTRODUCTION

Belt drivers are very important components in many different machines. They are

Inexpensive and low maintenance making them ideal for certain uses. In the write

Configuration they can also be used in gear boxes of cars to provide continuously variable gear ratios. This technique is becoming more common in high end sports cars which have “flapy paddle” gear shifting technology. They are also used for far simpler uses such as (sticking with the car) driving the fan in the front of an engine. However for all these purposes there is one large limiting factor which is that belt drives tend to slip. The amount of slip depends on several factors, the coefficient of friction between the belt and the pulley, the belt profile and the torque transmitted.

APPARATUS

Steel cylinder with wheels rotatable by wheels.

Metre rule.

rope

DESCRIPTION OF EXPERIMENTAL SETUP

PROCEDURE

Using the equipment shown above

The belt was fixed in the v- groove and the peg was fixed into the top-most hole to give the angle of lap Ѳ, with the spring balance connected in between.

Weights are hung on the other side of the belt and T1 is recorded from the spring balance.

The handle was then rotated slowly and steadily and T2 was taken from the spring balance.

The process was repeated for the several values of the angle of lap Ѳ, and T1 and the corresponding value of T2 were also found.

The angle of lap was measured using a rule and thread.

THEORY

Let

T1 = tension in the belt where the weight is hanged

T2 = tension recorded by the spring balance on the other end

ѳ = the angle of lab or contact

= co-efficient between the belt and the pulley

The ratio of the two tensions may be found by considering an elemental piece of the belt MN subtending an angle at the centre of the pulley B as shown below. Forces keeping MN in equilibrium include:

- Tension T in the belt at M acting tangentially,

- Tension T + in the belt at N acting tangentially,

- Normal reaction R acting outward at P (p is at the middle of MN)

- Friction force (F) = R acting at right angle to R and in the opposite direction of the motion of the pulley.

From the diagram above and by Pythagoras theorem, R=Tsinδθ2

Taking,T=(T+δT), » R=T+δTsinδθ2

Since the angle is too small,sinδθ2=δθ2

R=T×δθ2+T×δθ2δT×δθ2

=T×δθ+δT×δθ2

Since δT×δθ2 is negligible, it implies that R=T×δθ……………..1

Resolving vertical forces from diagram,

F=T+δTcosδθ2-Tcosδθ2

Also, since δθ is very small, cosδθ2=1 thus, reduces to unity.

It implies that, F=T+δT-T

=δT

But F=µR

Implying that, µR=δT

R=δTµ…………..2

Equating (1) and (2),

T×δθ=δTµ

Therefore, µδθ=δTT

Integrating the above between limits T1 and T2,

Implies that T2T1δTT=µδθ

lnT1T2=µθ

Therefore, T1T2=eµθ

For a flat belt,T1T2=eµθ

Diagram of the V-belt in the v groove

Let

RN = Normal reaction between belt

2 = angle of groove

= co-efficient of friction between belt and pulley

R = total reaction in the plane of groove

Resolving vertical forces on diagram, R=Rnsinα+Rnsinα

=2Rnsinα

⇒Rn=R2sinα=R2cosecα

But friction =2µRn

=2µ×R2cosec

Rn=R2cosecα

= µR×cosecα

=Rµcosecα

From the above comparing with flat belt,

⇒T1T2=eθµcosecα

=eµθ cosecα

=eµ1θµ1=µcosecα   Therefore, µ1=µsinα

Calculations

There are four different grooves in the setup. There are three (3) v-grooves and one (1) flat groove. The three v – grooves are denoted 1st groove, 2nd groove and 3rd groove. The last one is a flat pulley.

The circumferences of the various grooves are listed below:

Groove 1 = 27.6cm

Groove 2 = 29.8cm

Groove 3 = 29.8cm

Flat pulley = 31.00cm

FOR PEG 1

Angle of lap for groove 1 = 8   29×360°=99.31°

Angle of lap for groove 2 = 6.330×360°= 75.6°

Angle of lap for groove 3 = 6.130.5×360°= 72.0°

Angle of lap for flat pulley   = 5.930.7×360°= 69.19°

FOR PEG 2

Angle of lap for groove 1 = 6.8   29×360°=84.41°

Angle of lap for groove 2 = 730×360°= 84°

Angle of lap for groove 3 = 7.530.5×360°= 88.52°

Angle of lap for flat pulley   = 830.7×360°= 93.81°

FOR PEG 3

Angle of lap for groove 1 = 11   29×360°=136.55°

Angle of lap for groove 2 = 1230×360°= 144°

Angle of lap for groove 3 = 1330.5×360°= 153.44o

Angle of lap for flat pulley   = 1430.7×360°= 164.16°

Solving for µ from

T1T2=eµθ, ⇒µ=lnT1T2θ

TABLE OF RESULTS

TABLE OF VALUES FOR PEG 1

| Weight | Groove 1 |  |  | Groove 2 |  |  | Groove 3 |  |  | Flat |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T1 | T2 | µ | T1 | T2 | µ | T1 | T2 | µ | T1 | T2 | µ |
| 2 | 3 | 8 | -0.246 | 4 | 6 | -0.134 | 4 | 5 | -0.077 | 3 | 5 | -0.184 |
| 4 | 5 | 15 | -0.275 | 5 | 10 | -0.228 | 5 | 9 | -0.203 | 5 | 8 | -0.169 |
| 6 | 8 | 17 | -0.189 | 7 | 14 | -0.228 | 7 | 11 | -0.156 | 7 | 11 | -0.162 |
| 8 | 10 | 19 | -0.161 | 10 | 18 | -0.194 | 9 | 14 | -0.152 | 9 | 14 | -0.159 |
| 10 | 13 | 23 | -0.143 | 11 | 21 | -0.213 | 12 | 18 | -0.140 | 11 | 17 | -0.157 |

GROOVE 1 PEG 1GROOVE 2 PEG 1

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 0.903 |
| 4 | 0.699 | 1.176 |
| 6 | 0.903 | 1.230 |
| 8 | 1.000 | 1.279 |
| 10 | 1.114 | 1.362 |

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.602 | 0.778 |
| 4 | 0.699 | 1.000 |
| 6 | 0.845 | 1.146 |
| 8 | 1.000 | 1.255 |
| 10 | 1.041 | 1.322 |

GROOVE 3 PEG 1FLAT PULLEY PEG 1

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.602 | 0.699 |
| 4 | 0.699 | 0.954 |
| 6 | 0.845 | 1.041 |
| 8 | 0.954 | 1.146 |
| 10 | 1.079 | 1.255 |

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 0.699 |
| 4 | 0.699 | 0.903 |
| 6 | 0.845 | 1.041 |
| 8 | 0.954 | 1.146 |
| 10 | 1.041 | 1.230 |

TABLE OF VALUES FOR PEG 2

| Weight | Groove 1 |  |  | Groove 2 |  |  | Groove 3 |  |  | Flat |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T1 | T2 | µ | T1 | T2 | µ | T1 | T2 | µ | T1 | T2 | µ |
| 2 | 3 | 10 | -0.355 | 3 | 8 | -0.291 | 3 | 6 | -0.195 | 3 | 6 | -0.184 |
| 4 | 5 | 17 | -0.361 | 6 | 13 | -0.230 | 5 | 10 | -0.195 | 5 | 7 | -0.089 |
| 6 | 9 | 22 | -0.264 | 8 | 18 | -0.240 | 7 | 13 | -0.174 | 7 | 13 | -0.164 |
| 8 | 10 | 25 | -0.270 | 10 | 21 | -0.220 | 10 | 16 | -0.132 | 9 | 16 | -0.153 |
| 10 | 12 | 25 | -0.216 | 11 | 25 | -0.243 | 12 | 20 | -0.144 | 11 | 20 | -0.159 |

GROOVE 1 PEG 2GROOVE 2 PEG 2

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 1.000 |
| 4 | 0.699 | 1.230 |
| 6 | 0.954 | 1.342 |
| 8 | 1.000 | 1.398 |
| 10 | 1.079 | 1.398 |

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 0.903 |
| 4 | 0.778 | 1.114 |
| 6 | 0.903 | 1.255 |
| 8 | 1.000 | 1.322 |
| 10 | 1.041 | 1.398 |

GROVE 3 PEG 2                                                               FLAT PULLEY PEG2

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 0.778 |
| 4 | 1.000 | 0.845 |
| 6 | 0.845 | 1.114 |
| 8 | 0.954 | 1.204 |
| 10 | 1.041 | 1.301 |

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 0.778 |
| 4 | 0.699 | 1.000 |
| 6 | 0.845 | 1.114 |
| 8 | 1.000 | 1.204 |
| 10 | 1.079 | 1.301 |

TABLE OF VALUES FOR PEG 1

| Weight | Groove 1 |  |  | Groove 2 |  |  | Groove 3 |  |  | Flat |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T1 | T2 | µ | T1 | T2 | µ | T1 | T2 | µ | T1 | T2 | µ |
| 2 | 3 | 14 | -0.281 | 3 | 10 | -0.208 | 3 | 8 | -0.160 | 3 | 7 | -0.128 |
| 4 | 6 | 22 | -0.237 | 5 | 16 | -0.201 | 5 | 12 | -0.142 | 5 | 12 | -0.133 |
| 6 | 8 | 25 | -0.208 | 8 | 20 | -0.158 | 7 | 17 | -0.144 | 7 | 16 | -0.125 |
| 8 | 9 | 25 | -0.186 | 9 | 25 | -0.177 | 10 | 21 | -0.120 | 9 | 19 | -0.113 |
| 10 | 11 | 25 | -0.150 | 11 | 25 | -0.143 | 11 | 25 | -0.133 | 11 | 21 | -0.098 |

GROOVE 1 PEG 3

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 1.146 |
| 4 | 0.778 | 1.342 |
| 6 | 0.903 | 1.398 |
| 8 | 0.954 | 1.398 |
| 10 | 1.041 | 1.398 |

GROOVE 2 PEG 3

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 1 |
| 4 | 0.699 | 1.204 |
| 6 | 0.903 | 1.301 |
| 8 | 0.954 | 1.398 |
| 10 | 1.041 | 1.398 |

GROOVE 3 PEG 3FLAT PULLEY PEG 3

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 0.903 |
| 4 | 0.699 | 1.079 |
| 6 | 0.845 | 1.230 |
| 8 | 1.000 | 1.322 |
| 10 | 1.041 | 1.398 |

| Weight | log T1 | log T2 |
| --- | --- | --- |
| 2 | 0.477 | 0.845 |
| 4 | 0.699 | 1.079 |
| 6 | 0.845 | 1.204 |
| 8 | 0.954 | 1.279 |
| 10 | 1.041 | 1.322 |

Slope = 1.397            Intercept = -0.825

Slope = 0.85          Intercept = -0.0957

Slope = 0.873

Intercept = -0.054

Slope = 1.061          Intercept = -0.263

Slope = 1.45          Intercept = -1.004

Slope = 1.152          Intercept = -0.541

Slope = 1.188         Intercept = -0.462

Slope = 0.781

Intercept = -0.324

Slope = 1.931

Intercept = -1.750

Slope = 1.342

Intercept = -0.877

Slope = 1.163

Intercept = -0.567

Slope = 1.152

Intercept = -0.517

| Peg/groove | Angle (radians) | µ | Log(T1)/log(T2) (mean) |
| --- | --- | --- | --- |
| 1/1 | 1.733 | -0.203 | 0.734 |
| 1 / 2 | 1.319 | -0.199 | 0.763 |
| 1/3 | 1.257 | -0.146 | 0.822 |
| 1/flat | 1.208 | -0.166 | 0.812 |
| 2 / 1 | 1.473 | -0.293 | 0.688 |
| 2/2 | 1.466 | -0.245 | 0.716 |
| 2/3 | 1.545 | -0.168 | 0.780 |
| 2/flat | 1.637 | -0.150 | 0.773 |
| 3/1 | 2.383 | -0.212 | 0.646 |
| 3/2 | 2.513 | -0.177 | 0.668 |
| 3/3 | 2.678 | -0.140 | 0.703 |
| 3/flat | 2.865 | -0.119 | 0.719 |

COMMENTS

The accuracy of the experiment  could  be seen to be moderate since some are determined to be more than 1 and others less than 1. This could be attributed to the spring balance used to measure the tensions T1 and T2. The spring might have been plastically deformed.

When the belt is not slipping, the coefficient of friction should be 1.

The value of  obtained in the experiment is 0.063 .

Percentage accuracy % = 6.3% (taken from the slope obtained, 0.063, compared to the standard slop, 1)

Published value of μ for steel in contact with leather is 0.4 to 0.6

CONCLUSION

Belt drives use the friction between the pulley and the belt around the arc of the contact to transmit torque. The belt is initially stretched with an initial tension between the two pulleys.

For efficient performance, the belt is supposed to touch the bottom of the groove. But in this experimental technique, the belt touches the sides of the grooves and this is a limitation of the technique. I suggest that, a belt with a smaller diameter is used in performing the experiment

REFERENCE

Lab manual

Wikipedia search engine

221 Friction Proced spr2006