KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY

FACULTY OF AGRICULTURAL AND MECHANICAL ENGINEERING

DEPARTMENT OF AGRICULTURAL ENGINEERING

MECHANICAL ENGINEERING LAB III (ME 295)

CALIBRATION OF A LEAF SPRING

GROUP P

Topics:

- Springs

- Leaf spring

Names of members:

NTIM FOSU KWABENA			1419707

NIMO OSEI KOFI				1419607

MENSAH -  DEBRAH PATRICK	1419507

Objective: A comparison the theoretical and experimental spring stiffness.

Theory:

Springs are generally materials that are able to absorb and store energy, releasing it when needed. They act as shock absorbers such as in vehicles and spring of buffers of a railway rolling-stock. They are also used in the restoration of mechanisms to their original configuration against the action of an external force.

Some properties of a spring that makes it useful in engineering

These properties include:

- Its capacity to store energy.

- Its natural frequency of vibration.

- Its ability to reduce stress on a material so that it’s working stress is not exceeded.

A leaf spring is a long, flat, thin, and flexible piece of spring steel or composite material that resists bending. Two or more of these pieces are mounted on each other joined by a clip. The clips are installed to assist the leaf spring suspension when exposed to shock and rebound loads and prevent the spreading of the leaves.Example; most heavy duty vehicles today use leaf springs on their axle, mounted perpendicularly to support the weight of the vehicle. This system also contributes to the general stability of the vehicle. On the basis that springs are able to store energy, leaf springs are also used as shock absorbers.

Deflection

n = number of plates

t = plate thickness

b = plate width

L = distance between the spring eyes

Methodology

Apparatus: Leaf spring, venier caliper, dial gauge, tape measure,

Procedure:

- L is obtained using the tape measure, b and t are obtained using the venier caliper and n is also obtained by counting the leaves.

- The spring is set to properly stand on the bench.

- The dial gauge is then fixed with its magnetic base firm on the bench and the plunger fitting well into the center bolt.

- Loads from 0 to 80lb are then hung with 10lb increments, and for each load values of deflection are recorded.

- The spring is then unloaded with 10lb intervals and for each one, deflections are recorded.

| LOADING (W) | LOADING | UNLOADING |
| --- | --- | --- |
| 10 | 10 | 72 |
| 20 | 45 | 136 |
| 30 | 96 | 195 |
| 40 | 154 | 256 |
| 50 | 208 | 321 |
| 60 | 257 | 343 |
| 70 | 313 | 359 |
| 80 | 368 | 368 |

Table in S.I UNIT

| LOAD | LOADING | UNLOADING |
| --- | --- | --- |
| 44.5 | 0.254 | 1.800 |
| 89 | 1.143 | 3.400 |
| 133.5 | 2.400 | 4.875 |
| 178 | 3.912 | 6.400 |
| 222.5 | 5.200 | 8.025 |
| 267 | 6.425 | 8.575 |
| 311.5 | 7.825 | 8.975 |
| 356 | 9.200 | 9.200 |

The theoretical values

Putting in values of W for each increment, the table below is obtained.

| Load/ kgms-2 | Loading/ m (x 10-3) |
| --- | --- |
| 0 | 0 |
| 44.5 | 1.46 |
| 89 | 2.92 |
| 133.5 | 4.39 |
| 178 | 5.84 |
| 222.5 | 7.30 |
| 267 | 8.76 |
| 311.5 | 10.22 |
| 356 | 11.68 |

Precautions

- The plunger is set to fall right onto the center bolt.

2.     Care is taken so that no one leans on or nothing disturbs the bench.

3.     The weight is allowed to settle before any reading is taken.

Observation & Comments.

- The area under the loop represents the amount of work which has been lost to the surroundings.

- The area should be as small as possible because reducing it means reducing all sorts of energy loses thereby boosting the efficiency of the component.

- The area could be reduced through the reduction of friction. Friction could be reduced by lubrication, polishing of the rubbing surfaces and also by the use of rollers and ball bearings.