KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY

FACULTY OF AGRIC AND MECHANICAL ENGINEERING

DEPARTMENT OF MECHANICAL ENGINEERING

NAME: ANANE Clifford Owusu

INDEX NUMBER: 2666108

GROUP F

MECHANICAL ENGINEERING LABORATORY (ME295)

21ST SEPTEMBER 2009

FLUID MECHANICS LAB

CALIRBATION OF A PRESSURE GUAGE

INTRODUCTION

A pressure gauge is a device used to determine pressure from an applied load. A pressure gauge is also used to determine atmospheric pressure.

OBJECTIVE

Through applying a number of weights over a fixed surface area of the piston, the objective of the experiment is to calibrate a pressure gauge.

APPARATUS

A bourdon pressure gauge was used in carrying out the experiment. Other apparatus used include a piston of area 315mm2 and mass of 1kg, a cylinder, a tube which connected the cylinder to the pressure gauge and a dead weight.

PROCEDURE

The piston was removed from the cylinder to allow the cylinder be filled with water until the water overflowed. It was observed that there was air bubbles trapped in the tube and so the tube was tilted and tapped gently until all the air bubbles were cleared. After making sure that the cylinder was positioned vertically, it was then refilled with water. The piston of mass 1kg was then placed into the cylinder and the corresponding pressure reading recorded. Thereafter, weights were added in convenient increments of 5kg with the corresponding value on the pressure gauge recorded with each increment. This was done for 10 increments. The same procedure was repeated for the unloading of the dead weights with the corresponding pressure recorded as decreasing pressure. The values increasing and decreasing pressure against their corresponding mass were tabulated along with the values of the true pressure which was obtained by calculation as:

P=m×gA

Where P represents the true pressure,

M represents the mass of the dead weight applied,

G represents the acceleration due to gravity acting on the piston and the dead weight and

A represents the total surface area of the pressure gauge.

Below is a graph of true pressure, increasing pressure and decreasing pressure.

CONCLUSION

It was observed from the experiment that the values of the true pressure obtained were not consistent with the values of the increasing and decreasing pressures, which was as a result of the spring used in the pressure gauge.