PROPERTIES OF ENGINEERING SOILS

Engineering Definition of Soil

For engineering purposes, a soil is considered to be any loose sedimentary deposit such as gravel, sand, silt or clay or a mixture of these materials.  It should not be confused with the geological definition of soil, which is the weathered organic material on the surface or top soil.  The topsoil is generally removed before any engineering project can be carried out.  The characteristics of the topsoil are of importance to the agriculturist only.  Usually the top soil contains organic matter which decomposes with respect to time and changes the behavior of the soil.  For this reason, it is not good for engineering construction works and must be removed prior to any such work.

Definition of Particle Size and Shape

The process of weathering and the action of transportation and deposition produces individual particles of soil which vary widely in size and shape.  The size of a particle in a soil deposit has a fundamental influence on the properties and engineering behavior of the deposit.  Therefore particles of soil are described according to their size using terms such as gravel, sand, silt or clay sizes.  However, there is no universally agreed standard for use of these terms in the definition of particle size.  The more acceptable classification are based on the British, American and the Unified classification systems as shown in the table below:

Table 1. Soil classification

| Particle description | Particle Size (mm) |  |  |  |
| --- | --- | --- | --- | --- |
|  | British Standard (BS) | AASHTO | ASTM | Unified |
| Gravel | 60 – 2 | 75 – 2 | >2 | 75 – 4.75 |
| Sand | 2 – 0.06 | 2 – 0.05 | 2 – 0.075 | 4.75 – 0.075 |
| Silt | 0.06 – 0.002 | 0.05 – 0.002 | 0.075 – 0.005 | <0.075 |
| Clay | <0.002 | <0.002 | <0.005 |  |

AASHTO – American Association of State Highways and Transportation Officials; ASTM – American Society for Testing of Materials

The engineering behavior of a soil deposit is fundamentally dependent on the forces that act at the areas of contact between individual particles.  These are mainly gravitational forces related to the mass and therefore approximate to the volume of the particles and surface forces derived from electrochemical activity on the surface of the soil particle.  Clay mineral particles are not only small because of their plate-like shape but have a large surface area to volume ratio and consequently, they experience surface forces which predominate over the mass derived gravitational forces.  The dominant influence of these surface forces result in a coalition or stiction of the clay particles.  These are called cohesive forces.  Therefore any soil that contains a certain amount of clay exhibits the properties of cohesion.  On the other hand, the surface forces that act in the gravel and sand particles are negligible and therefore are classified as cohesionless or granular soils.

In a typical engineering soil, the individual particles which may vary considerably in size and shape form a framework of solid material and between the individual particles is a system of interconnected spaces.  The framework of soil particles is referred to as the soil structure and the spaces between particles as voids or pores.

Below the water table, the pores will generally be completely filled with water while above the water table, the pores will partly be filled with water and air.  When the pores are completely filled with water, the soil is said to be saturated.  If they are filled with both water and air, the soil is said to be unsaturated.  If the pores are completely filled with air, the soil is said to be air-dry.  Therefore, strictly speaking, all soil are said to be not only porous but permeable and this characteristic of permeability fundamentally influences the engineering behavior of a soil deposit in many practical situations.

Physical Properties of Soils

|  | SOIL |  |
| --- | --- | --- |
| volume of air = Va | Air | Ma = mass of air |
| volume of water = Vw | Water | Mw = mass of water |
| volume of solids = Vs | Solids | Ms = mass of solids |
| Total volume of soil = Va + Vw + Vs = V |  | M = Ma + Mw + Ms = total mass of soil |

Figure 2.  Schematic diagram of a soil mass

From the diagram above,

Soil Water Content

- Mass water content of the soil (θm ) is defined as:

θm = MwMs

- Water content by volume (θv ) is defined as:

θv = VwV

Soil Density

- Soil wet bulk density (ρwb ):

ρwb = MV; where M = Mw + Ms

- Soil dry bulk density (ρd ):

ρd = MsV

- Soil particle density (ρs ):

ρs = MsVs; soil particle density is about 2.65 g/cm3for all soils.

- Soil saturated density (ρsat):

ρwb = MV; where M = Mw + Ms

- Submerged density (ρsub ):

ρsub = ρsat- ρw

Soil porosity, void ratio, degree of saturation and air voids content

- Porosity (p):

p = VvV

- Void ratio (e):

e = VvVs

- Degree of saturation (θs ):

θs = VwVv

If θs =1;the soil is saturated;  θs =0;the soil is air dry

- Air voids ratio (θa )

θa = VaV

Quantity of water present in the soil

- Depth of water (d) present in a certain depth (profile) of soil (Z):

d= θvZ

Relationships between soil physical properties

| 13. | θv= ρdθm | θm= gravimetric water content θv= volumetric water content θs= degree of saturation θa= air voids content ρd= dry bulk density ρwb= wet bulk density ρw= density of water Gs= specific gravity of soil particles p= porosity e= void ratio |
| --- | --- | --- |
| 14. | θa=p- θv |  |
| 15. | θa=p (1- θs) |  |
| 16. | θa= e- Gsθm1+e |  |
| 17. | ρwb= ρd(1 + θm) |  |
| 18. | ρd= Gsρw1+e |  |
| 19. | ρd= Gsρw(1- θa)1+ Gsθm |  |
| 20. | p= e1+e |  |
| 21. | θs= θm+Gse |  |

Problems

- A farm road embankment is constructed of clay fill compacted to a bulk density of 2.05 Mg/m3 at water content of 18 % by mass.  The specific gravity of the soil particles is 2.70.  Calculate the porosity, degree of saturation, air content and dry bulk density of the clay material.

- A road embankment is constructed of fill material compacted to a density of 2.15 Mg/m3 at a water content of 11.5 %.  The specific gravity of the soil grade is 2.70.  Calculate the dry bulk density, porosity, degree of saturation and air void content of the field material.

- An investigation at a particular site revealed a deposit of sand overlying bedrock.  A sample of sand taken from above the water table had a total mass of 2205 g and was found to have occupied in its natural state a volume of 1125 cm3.  After oven-drying the mass of the sample was 1970 g and its particle specific gravity was found to be 2.65.  a)  For the sand above the water table, calculate the wet bulk density, water content by mass, void ratio, degree of saturation and air voids content. b) For the sand below the water table, calculate water content by mass and the submerged density of the soil.