TECHNICAL SYSTEMS

A technical system may consist of one or more elements (parts) such as shaft, bearing, gear, belt, capacitors, resistors, vanes etc.. A technical system has a boundary that may be opened or closed to flow of material, energy or signal (open system and closed system). A group of elements can also form a sub-system within the main-system. Such an example is the gear-box of an automobile.

A technical system can also be classified according to the type of energy flow, such as mechanical system (eg. crane), electrical system (eg. electric motor), hydraulic system (eg. hydraulic pump),

DESIGN OF MACHINE ELEMENTS

Basically, machine elements are design to carry loads, and therefore analysis of loads is essential in machine element design against strength failure. The process can be seen to involve two parallel flows as shown below.

The first step is to accurately determine the load.

CRITICAL SECTION

To determine when a machine element will fail, the designer evaluates the stress, strain, and strength at the critical section. The critical section, or the location in the design where the maximum internal stress is developed and failure is most likely, is often not intuitively known a priori.

In order to determine the critical section and loadding, the designer must:

- Consider the external loads applied to a machine (e.g., an automobile),

- Consider the external loads applied to an element within the machine (e.g., a cylindrica1 rolling-element bearing),

- locate the critical section within the machine element (e.g., the inner race), and

- determine the loading at the critical section (e.g., contact stresses)

The critical section will often occur at:

- locations of geometric nonuniformity, such as where a shaft changes its diameter along a fillet,

- locations where load is applied or transferred, and

- locations where the geometry is most critical.

MECHANICAL SYSTEMS

An object posses three different types of motion on a plane The plane can be either the x-y, x-z, or y-z plane. The object can be slided in two directions and as well be rotated about an axis that is perpendicular to the plane formed by the sliding axes.

For example, a body can slide in the x or y-axis of x-y plane, and rotate about the z-axis that is perpendicular to the x-y plane. This body is then said to have three degrees of freedom (three conditions of equilibrium) on the x-y plane.

A support restricts the possibilities of a motion, i.e. the number of degrees of freedom:

|  | In a Plane (eg. x-y plane) | In Space |
| --- | --- | --- |
| Degrees of freedom | - Translation in x-, y- axes - Rotation about z-axis | - Translation in x-, y- z-axes - Rotation about x-, y- z-axes |
| Number of degrees of freedom | 3 | 6 |
| Forces and moments present |  |  |
| Conditions of equilibrium |  | - , - , - , |

|  |  | Degree of freedom |  |  | Support reaction |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sliding | Rotation | Total | Forces | Moments | Total |
| Pin/hinge |  | 0 | 1 | 1 | 2 | 0 | 2 |
| Sliding |  | 1 | 1 | 2 | 1 | 0 | 1 |
| Rigid |  | 0 | 0 | 0 | 2 | 1 | 3 |

ANALYSIS OF STRUCTURES

Static Quantities

Static quantities are quantities associated with forces, intensity of forces (e.g. pressure, traction, and stress), or resultants of forces (e.g. moment and torque). Three basic static quantities of primary interest in the analysis of structures are applied loads, support reactions, and internal forces as shown below.

Applied loads represent prescribed forces, intensity of forces, resultants of forces, or in combination that are exerted to the structure by surrounding environments. It is emphasized that applied loads are known a priori before the analysis procedure is carried out.

Support reactions represent unknown forces, intensity of forces, resultants of forces, or in combination that are exerted to the structure by (idealized) supports to maintain its equilibrium and stability under the action of applied loads or other excitations. The support reactions are unknown a priori in nature and can only be obtained from static analysis.

Internal forces represent intensity of forces (e.g. stress) or resultant forces (e.g. axial force, shear force, bending moment, torque) induced at a material point or over a particular section of the structure under the action of applied loads or other excitations. The internal forces are unknown a priori and can only be obtained from static analysis.

Static equilibrium

Equilibrium condition of a body is a statement of conservation of linear momentum and angular momentum of that body. More precisely, the body is in equilibrium if and only if the linear momentum and angular momentum are conserved for any part of the body (the entire body can also be considered as a part of that body). For a two dimensional body on a region on the X-Y plane, the body must satisfy the following three equations:

FX = 0

FY = 0

MAZ = 0

where A is an arbitrary point used for computing the moment about the Z-axis and {X, Y, Z; O} denotes a reference Cartesian coordinate system

Method of structure partitioning

To determine the internal forces at any location of the structure, the consideration of equilibrium of certain parts of the structure is required in addition to that of the entire structure. From the fact that the structure is in equilibrium if and only if any part of the structure is in equilibrium, any portion of the structure resulting from partitioning of the structure must be in equilibrium with applied loads acting to that portion (i.e. reactions at supports present in that portion and internal forces exerted to that portion by the rest of the structure).

For instance, a rigid frame is partitioned into two parts by a fictitious cut at point B. The internal forces {FB, VB, MB} at point B appear in both FBDs.

Since each portion of the structure resulting from partitioning must be in equilibrium, several equilibrium equations are therefore provided in addition to the equilibrium equations set up on the entire structure. However, the number of unknowns associated with the internal forces appearing at the cuts also increases at the same time. Hence, locations of the cuts and the number of the cuts are important and must properly be chosen in order to ensure that the number of all unknowns does not exceed the number of available equilibrium equations.

Static Determinate and Indeterminate Structure

Statically determinate structures belong to a special class of structures whose reactions at all supports and the internal forces at any location of the structure can completely be determined from equilibrium equations.

Determination of Support Reactions

This section demonstrates the application of static equilibrium to compute all support reactions of external, statically determinate structures (note that statically determinate structures are also contained in this class of structures.

### Simultaneous Equations

### If you have a number of unknowns and there are an equal number of independent equations, then the unknown variables can be determined.

CHARACTERISTICS OF STRUCTURES

Truss

An idealized structure is termed a truss if and only if:

- all members are straight,

- all members are connected by pinned (or hinge or truss) joints,

- all applied loads are in terms of concentrated forces and they act only at joints, and

- all supports provides only constraints against translations.

Examples of trusses are shown schematically in Figure below. Note that concentrated moments are not allowed to be applied to any joint of the truss since they provide no rotational constraint.

Beams

An idealized structure is called a beam if and only if:

- all members are straight and form a straight-line-configuration structure,

- all members are generally connected by beam joints (full moment release is allowed for certain joints), and

- all applied loads must form a system of transverse loads.

Examples of beams are shown schematically below. Note that there is no restriction on the type of supports present in beams, i.e. roller supports, pinned supports, guide supports and fixed supports are allowed. Note, however, that the component of support reactions in the direction of the beam axis, if exists, can be ignored since all applied loads are transverse loads.

Frames

An idealized structure is called a planar frame if and only if :

- all members form a two dimensional structure,

- members are connected by rigid or frame joints (full or partial moment releases are allowed for certain joints and this can be viewed as rigid joints supplying by moment releases), and

- applied loads form a system of general two-dimensional forces and moments (i.e. it includes both transverse and longitudinal loads).

Examples of planar frames following the above definition are below. Note that there is no restriction on the type of supports present in frames, i.e. roller supports, pinned supports, guide supports and fixed supports are allowed.

### Number of Unknowns:

### The figure below shows the most common support symbols and the independent reaction components for each. Column rc shows the corresponding number of independent reaction components.

Fig.:	Reaction Components

r:	The number of independent reaction components for the entire structure, r=∑rc.

Members of a structure are those portions that span between supports, or joints (connections to other members), or free ends. Each member has exactly two joints at either end. A point in a member can experience a maximum of three internal force components (normal force (N), shear force (V) and bending moment (M)).

For truss:

For beam:

For frame:

Then we have, as the number of unknowns:

### Number of Equations

To determine the number of independent equilibrium equations those are available to solve for the unknowns:

j :	The number of joints in the structure.

c :	The number of internal release conditions.

A condition is a constraint that specifies that an internal force must be a particular numerical value (almost always zero) at a point. An example is an internal hinge or pin that constrains the bending moment to 0.

It is possible to write  equations of equilibrium at each joint, and one more for each C condition. We state without proof, then, that the number of independent equilibrium equations is:

A brief summary of guidelines for determining support reactions is given below:

- Identify type of all supports

- Determine the number of unknown support reactions (ra)

    - Determine the number of independent equilibrium equations that can be set up for the entire structure ()

    - Determine the number of additional static conditions that can be set up without introducing new static unknowns ();

    - If ra > net + ncr, the structure is externally statically indeterminate. The support reactions cannot completely be obtained by using only equilibrium equations. If ra = net + ncr, the structure is externally statically determinate and support reactions can be determined

    - If ra = net, all support reactions can be obtained from equilibrium of the entire structure:

- sketch a free body diagram (FBD) of the entire structure,

- write down all independent equilibrium equations, and

- solve for unknown support reactions

    - If ra > net, all support reactions cannot be obtained from equilibrium of the entire structures alone and the subsequent steps are as follows:

- introduce suitable fictitious cuts; in general, fictitious cuts are made at the internal releases present within the structure (e.g. hinges, shear releases, axial releases),

- ensure that the number of fictitious cuts is sufficient to supply adequate number of equations to solve for all unknowns (i.e. all support reactions and extra unknowns associated with the internal forces induced at the cuts),

- sketch a free body diagram of the entire structure and free body diagrams of parts resulting from the cuts,

- write down all independent equilibrium equations for the entire structure and for parts resulting from the cuts, and

- solve for unknown support reactions.

A structure is internally statically determinate if all member forces (normal, shear and bending moments) can be completely determined using only the equations of statics.

The degree of indeterminacy of a structure is the excess number of unknown quantities (reactions and internal forces) over the number of independent equations of equilibrium available to solve for them. If the degree of indeterminacy is greater than 0, the structure is statically indeterminate. If it is equal to 0, the structure is statically determinate, but only if it is stable.

If the degree is less than 0, then the structure is unstable (see below), and its statical determinacy is not very relevant.

Example 	Determine all support reactions of a rigid frame shown below

Solution The entire structure is constrained at points A and B by a pinned support and a roller support, respectively; thus, the total number of support reactions is ra = 2 + 1 = 3. The number of independent equilibrium equations for the entire two-dimensional rigid frame is net  = 3. Since ra  = net, the structure is externally, statically determinate and all support reactions can be obtained by considering equilibrium of the entire structure. Free Body Diagram (FBD) of the entire structure is given below where {RAX, RAY} and RBY are unknown support reactions at point A and point B, respectively.

[FX = 0]   :	RAX + P = 0, 	RAX = –P	One unknown (RAX)

[FY = 0]   :	RAY + RBY – 4P = 0,	RAY = P 	One unknown (RAY)

[MA = 0]  :	(RBY)(2L) – PL – (P)(L) – (4P)(L) = 0	RBY = 3P	One unknown (RBY).

Example 	Determine all support reactions of a beam shown below

Solution 	Since ra = 2 + 1 + 1 = 4, considering individual members, m = 2, j = 3, c = 2, then DI = (4 + 2(2)) – (2(3) + 2) = 0. Thus, the structure is statically determinate and all support reactions can be determined from static equilibrium.

However, the number of independent equilibrium equations for the entire beam is net = 2 < ra; thus, the support reactions cannot be obtained by considering only equilibrium of the entire structure:

[MA = 0]  :	RAM + (RCY)(2L) + (REY)(4L) – (qL)(L/2) – (qL)(3L) = 0

RAM + 2RCYL + 4REYL = 7qL2/2	3 unknown

[FY = 0] :	RAY + RCY + REY – qL – qL = 0

RAY + RCY + REY = 2qL,	1 unknown

To clarify this, let us sketch the FBD of the entire beam as shown below.

Two additional equations associated with the presence of two moment releases or hinges at points B and D, i.e. M = 0 at B and M = 0 at D, must be employed.

By introducing two cuts, one at point B and the other at point just to the right of point D called DR, the original structure is decomposed into three parts and the FBD of each part is shown below. Note that the cut is not made exactly at point D due to the application of a concentrated load at point D and to avoid the question on how to distribute this concentrated load to the left and right parts of the point D. For this particular choice of the cut (cut at point DR), the concentrated load P = qL appears at the point DR of the FBD of the middle part.

The left portion contains three unknowns, i.e., {RAM, RAY, VB}; the middle portion also contains three unknowns, i.e., {RCY, VB, VDR}; and the right portion contains only two unknowns, i.e., {VDR, REY}. The total number of unknowns (including all support reactions and the shear forces appearing at the cuts) now becomes six, i.e., {RAM, RAY, RCY, REY, VB, VDR}. Two independent equilibrium equations can be set up for each individual portion and this leads to a set of six independent linear equations sufficient for determining all unknowns.

To avoid solving a system of six linear equations, we first consider the right portion in which the number of unknowns is equal to the number of independent equilibrium equations. By first applying equilibrium of moments about point E and then considering equilibrium of forces in Y-direction, we obtain

[ME = 0]  :	VDRL = 0, 	VDR = 0

[FY = 0] :	REY + VDR = 0,	REY = 0

Since the shear force VDR is already known, the number of unknowns for the middle portion now reduces to two, i.e., {RCY, VB}, and they can be solved by considering equilibrium of this portion as follows:

[MB = 0]    :	(RCY)(L) – (qL)(2L) – (VDR)(2L) = 0,	RCY = 2qL     Upward

[FY = 0]   : 	VB +RCY – qL – VDR = 0,	VB = –qL

Since the shear force VB  is already known, the number of unknowns for the left portion now reduces to two, i.e., {RAM, RAY}, and they can be solved by considering equilibrium of this portion as follows:

[MA = 0]     :	RAM – (qL)(L/2) – (VB)(L) = 0,	RAM = –qL2/2

[FY = 0]   :	RAY – qL – VB = 0,	RAY = 0

LOAD CLASSIFICATION.

Any applied load can be classified with respect to time in the following ways:

1.	Static load.	Load is gradually applied. and equilibrium is reached in a relatively shor time. The structure experiences no dynamic effects.

2.	Sustained load.	Load. such as the weight of a structure, is constant over a long time

3.	Impact load.	Load is rapidly applied. An impact load is usually attributed to an energy imparted to a system.

4.	Cyclic load.	Load varies and even reverses itself in direction and has a characteristic period with respect to time.

The load can also be classified with respect to the area over which it is applied:

1.	Concentrated load.	Load is applied to an area much smaller than the loaded member.

2.	Distributed load.	Load is spread over the entire area. An example would be the weight of a concrete bridge floor of uniform thickness.

Loads can be further classified with respect to location and method of application. Also. the coordinate direction must be determined before the sign of the loading can be established.

l.	Normal load.	The load passes through the centroid of the resisting section. Normal loads may be tensile or compressive. The sign convention has tensile load being positive and compressive load negative.

2.	Transverse Shear load.	The force P is assumed to be collinear with the transverse shear force. V. The separated bar in Figure (c) illustrates the action of positive shearing

- Bending load.	This commonly occurs when load is applied transversely to the longitudinal axis of the member.

- Torsion load.	This load subjects a member to twisting motion. The twist results in a distribution of shear stresses on the transverse cross section of the member.

Sign Convention

Members of a frame can possess different orientations, it is generally impossible to find a single reference Cartesian coordinate system with one of its axes directing along the axis of all members as in the case of beams. As a result, it is common to employ two different types of reference coordinate systems:

- Global coordinate system

The global coordinate system is a single coordinate system used as a reference of the entire structure. The global reference axes are labelled by X, Y, and Z with their directions following the right-hand rule.: the positive x-axis pointing in the direction of the first finger of our right hand, the positive y-axis pointing in the direction of our second finger and the positive z-axis pointing up in the direction of our thumb.

- Local coordinate system. The local coordinate system is a coordinate system defined for an individual member and its reference axes are labelled by x, y, and z. The local coordinate system for each member is commonly defined based on the geometry and orientation of that member. An example of global and local coordinate systems of a planar frame is shown below.

Figure 2.42: Global and local coordinate systems of a planar frame.

The sign convention and notations for support reactions of structures is denoted by the point of location and its direction with respect to the coordinate axis. They possess a positive direction if they direct along the positive coordinate axes. In the analysis for support reactions, it is common to assume positive support reactions in a sketch of the FBD. Once results are obtained, the actual direction of each reaction can be decided from their sign; specifically, if the computed reaction is positive, the assumed direction is correct but, if the computed reaction is negative, the actual direction is opposite to the assumed direction.

Figure : Schematic showing sign convention and notations of support reactions of a frame

For the internal force or member force of trusses, it is standard to follow notations and sign convention as described below The sign convention for the shear force and the bending moment for a frame member depend primarily on a choice of the local coordinate system

      - The shear force at a particular point A is denoted by a symbol VA and the shear force as a function of position x along the member in the x-direction is denoted by V(x). A shear force is positive if the force will cause a clockwise rotation of the freebody.

Positive shear force	Negative shear force

The shear force V shown on the right surface of the left section of Figure (c) above wil cause a clockwise rotation of that section. Thus, the shear force is positive.

On the left surface of the right section of Figure (c) the shear force is also positive since it will also cause a clockwise rotation of that section.

The shearing force (SF) at any section of a beam represents the tendency for the portion of the beam on one side of the section to slide or shear laterally relative to the other portion.

      - There are two sign conventions associated with bending moment:

Figure 2.3	Sign conventions used in bending:

(a) Positive moment leads to a tensile stress in the positive y-direction;

(b) Positive moment acts in a positive direction on a positive face.

Consider an element subjected to bending. This is shown below:

If an element of length  is considered before and after subjected to bending, the top length will be subjected to compression and therefore experience contraction.

This implies that the strain will vary linearly with y from the neutral axis. The upper half is compressed and will result in a negative tensile stress. Thus, M in the direction indicated must be negative. Considering our basic equation of bending:

For 	Since  is negative, M is negative for positive y. And therefore:

:	This follows that V is also negative for positive x. This agrees with the right side of case (a).

However, we will be considering the case where tension will be on the top surface, so we will adapt convention (a).

- Torsion load subjects a member to twisting motion. The twist results in a distribution of shear stresses on the transverse cross section of the member. The right-hand screw rule is applied to determine the positive direction.

- Positive shear force will cause clockwise rotation

- Positive bending moment will cause tension on the upper fibre.

The sign convention for bending moments used in stress analysis is arbitrary, and correct answers can be obtained for problems using either sign convention, as long as the sign convention is applied consistently within a problem.

Determination of Force, Bending Moment And Torque Diagrams

FREE-BODY DIAGRAM

The resulting internal stresses in a structure must not exceed the allowable stress limit. Therefore, it is essential to determine the resulting internal forces and moments of a structure under given external loads.

The free-body diagram (section principle) is applied to the partial system and the internal stresses at the cut-section can be determined. For a system, there should be a well defined boundary. This partial system is an open-system. The section principle says that part of a system of a body is in equilibrium so far as the total system is in equilibrium.

Method of sections

- Determine all support reactions

- Introduce a cut at point P and then separate the beam into two parts

- Choose one of the two parts that seems to involve less computation

- Sketch the FBD of a selected part; both shear force and bending moment are assumed a priori to be positive.

- Apply equilibrium of forces of the selected part in Y-direction; this yields the shear force at point P (VP). If the computed shear force is positive, the assumed direction is correct; otherwise, the actual direction is opposite to the assumed direction.

- Apply equilibrium of moments of the selected part about a point P; this yields the bending moment at point P (MP). If the computed bending moment is positive, the assumed direction is correct; otherwise, the actual direction is opposite to the assumed direction.

B

The diagram below shows a beam carrying loads W1 W2 and W3.. It is simply supported at two points where the reactions are R1 and R2. Assume that the beam is divided into two parts by a section XX.

Fig A:	Complete beam	Fig B:	FB Diagram of Left portion

The resultant of the loads and reaction acting on the left of AA is F vertically upwards (Fig A), and since the partial beam is in equilibrium, the resultant force to the partial beam at AA must be VA downwards (Fig B). VA is called the Shearing Force at the section AA

Where forces are neither in the lateral or axial direction they must be resolved in the usual way and only the lateral components are used to calculate the shear force.

In a similar manner to the example above, it can be seen that if the bending moments (BM) of the forces to the left of AA are clockwise, then the bending moment of the forces to the right of AA must be anticlockwise (Fig. A).

Fig A:	Complete beam	Fig B:	FB Diagram of Left portion

The free body of the left portion of the beam must however be in equilibrium when a counter moment acts on the section A-A as shown in Fig B.

Bending Moment at AA is defined as the algebraic sum of the moments about the section  of all forces acting on either side of the section.

Example

Classify each of the beams shown below as statically determinate or statically indeterminate. If statically indeterminate, report the number of degrees of indeterminacy. The beams are subjected to external loadings that are assumed to be known and can act anywhere on the beams.

Solution

Example:

Determine whether the sketch below deals with a statical determinate support.

Solution

Conditions of equilibrium:

This implies three conditions of equilibrium with three unknowns.

Example 2:

Three conditions of equilibrium:

One independent condition of symmetry:

From the equations: