AXLES AND SHAFTS

They do transmit rotary motions.

AXLES

An axle can be a stationary that carries rotating elements. They are basically subjected to bending and transverse forces. They are not often subjected to axial forces. However, they are never subjected to torsional forces. Rotating axle rotate with elements fixed on them. They are subjected to rotating (dynamic) bending.

Axles do not transmit torque and are mainly subjected to bending load due to the presence of transverse force, but effect rotary motion. They support rotary elements and can be either dynamically or statically loaded. This can be achieved through either:

- Mode of mounting (rotating or stationary axle)

- Applied force (static or rotating force).

The arrangement of the axle for the tension pulley in a belt transmission takes care of the permanent elongation of the belt.

Types of Loads

Static Loading for the Axle

Dimensioning of the axle will be based on the static strength values of the materials

Design formula:	.

Dynamic Loading for the Axle

An axle subjected to rotating bending moment:	From the figure above points “a” and “b” are experiencing maximum stresses under the same bending moment. However, if the point “a” rotates to point “c” there will be no stress on it (because the point “c” is on the neutral plane). Therefore, for the point “a” the stress-time diagram is shown below as it rotates:

Thus, for fatigue loading a bigger axle diameter may be required as compared to a statically loaded axle.

First calculate

nominal stress amplitude (it does not consider the effect of notches).

In order to take into consideration notch-effects, the stress fatigue factor,  must be introduce:

.

Compare  with fatigue strength values from data:

The Soderberg line is for pure tensile loading, where

Therefore:

for shear loading the end-points of the Soderberg line are:

for tensile loading the end-points of the Soderberg line are:

For pure tensile test:

This diagram is given for:

Standard specimen,

- Given size and

- Given surface quality.

Any deviation from this standard specimen must be considered:

- = size factor,

- = surface factor.

For the situation where failure is expected to be due solely to alternating component:

.

Where :

A shaft is normally designed to carry torque:

Dimensioning of Shaft

SHAFTS

A shaft is a rotating or stationary member usually having a circular cross section smaller in diameter than the shaft length. Mounted on it are elements such as gears, pulleys, belts, chains, cams, flywheels, cranks, sprockets, and rolling element bearings.

The loading on the shaft can be various combinations of:

- Bending (almost always fluctuating),

- Torsion (may or may not be fluctuating),

- Shock,

- Axial,

- Normal or transverse shear.

In practical applications, shafts are often stepped instead of a constant diameter in order to facilitate assembling of the other components.

Some of the main considerations in designing a shaft are:

- Strength, using yield or fatigue (or both) as a criterion; (low- and medium-speed shafts),

- Deflection, or

- Dynamics established by the critical speeds (high-speed shafts)

PROCEDURE FOR SHAFT DESIGN

A rough diameter, d, can be obtained for pre-dimensioning purposes:

A conservative design torsional stress,  is used, because:

- Shafts are loaded not only by torque but by other loads (eg bending), and

- Other factors are neglected.

With d obtained an assembled drawing of the shaft and its mounted components is accomplished. The nominal stresses induced on the shaft are determined and the critical sections are analysed for failure.

- Nominal stresses determination:

- Develop a free-body diagram by replacing the various machine elements mounted on the shaft by their statically equivalent load or torque components.

- Draw the torque, bending moment, axial force and shear force diagrams in the respective planes. The resultant internal forces and moment at any section along the shaft can be resolved. Example:

- Effective stresses (notch effect) Kt and Kf:

- Establish the location of the critical section, or the x location where the torque and the moment are the largest.

- Theories of failure:

- For ductile materials use the maximum-shear-stress theory (MSST) or the distortion energy theory.

- For brittle materials use the maximum-normal-stress theory (MNST), the internal friction theory (IFT), or the modified Mohr theory (MMT).

- Properties of material

- Factors (b1, b2)

- Load line

- Safety factor

- For normal conditions;

- For very clear conditions with respect to operational condition and reliable materials.

Given:	The power from the motor of a front-wheel-drive car is transmitted to the gear-box by a chain drive. The two chain wheels are of the same size. The chain is not pre-stressed, so the loose chain exerts no force. The safety factor is 4. The shaft is to be made of AISI 1080 steel. The chain transmits 100 kW of power at the chain speed of 50 m/s when the motor speed is 6000 rpm.

Find:	The appropriate shaft diameter by using the DET.

Solution:	Using  gives the force transmitted through the chain as:

The chain wheel radius is

The torque being applied by the chain is:

The maximum bending moment occurs at x=0.160m and has a value of 80 Nm. The torque is constant along the shaft, thus the critical section occurs at x=0.160m.

And

From Mohr’s circle, the principal stresses:

Von Mises stress:

The von Mises stress for biaxial stress state:

The DET predicts that failure will not occur if:

STATIC LOADING

Static loading implies:

- For a stationary point:	The force is stationary (with respect to the point) and steady (with respect to time), and

- For a stationary and steady force:	The point is stationary

- Bending Moment And Torsion

The force exerted on a shaft in the transverse direction (perpendicular to the shaft axis) produces a bending maximum stress of:

Similarly,

For plane stress (2-dimensional) states,:	Uni-axial

- Principal stresses from Mohr’s circle

- Principal shear stress:

- Distortion Energy Theory (DET)

The DET predicts it will not fail if the von Mises’ stress  satisfies the condition:

For

- Maximum-Shear-Stress Theory

The MSST predicts failure for biaxial stress state:

Case 1.

Case 2.

Case 3.	.

For the above situation of the shaft, case 3 applies:

And

Example:

Given:	An assembly has tensile belts and frictionless journal bearings at locations A and B. The yield strength of the shaft material is 500MPa and the safety factor is 2.

Find:	Determine the smallest safe shaft diameter by using both the DET and the MSST. Also provide a free-body diagram as well as moment and torque diagrams

x-z plane 					x-y plane

Solution:	From the moment diagrams the maximum moment is at point C

Using the DET, the smallest diameter is

Using the MSST gives (Applying case 3):

And

Since the torque is small relative to the moment, little difference exists between the DET and MSST predictions.

- Bending Moment, Torsion, and Axial Loading

If the axial load is added to the bending load:

- Principal stresses:

- Principal shear stress:

- Distortion Energy Theory (DET)

The DET predicts failure if the von Mises stress satisfies the condition:

Because of the addition of the axial load an explicit expression for the diameter cannot be obtained.

Maximum-Shear-Stress Theory

The MSST predicts failure for biaxial stress state:

Again, because of the addition of the axial load an explicit expression for the diameter cannot be obtained.

CYCLIC LOADING

Cyclic loading implies:

- For a stationary point:	The force may be either non-stationary (with respect to the point) or non-steady (with respect to time), and

- For a steady and static force:	The point may be non-stationary.

Here, a general analysis is derived for the fluctuating normal and shear stresses for ductile materials and appropriate equations are then given for brittle materials.

Ductile Materials

In this example of relating strength to shaft size, we opt for the:

- Maximum-shear-stress theory to predict the damaging stress, and

- Soderberg’s line to predict the significant strength.

The figure below shows the normal and shear stresses acting on a rectangular element of a ductile shaft. The figure on the right show stresses acting at an angle .

Designations:

- alternating

- mean

- fatigue stress concentration factor due to normal loading,

- fatigue stress concentration factor due to shear loading

The largest stress occurs when  are in phase, or when the frequency of one is an integral multiple of the other. Summing the forces tangent to the diagonal gives:

Making use of double angles reduces the above equation to:

Separating the mean and alternating components of stress gives the stress acting on the oblique plane as:

The Soderberg line is for pure tensile loading.

Therefore, for shear loading the end-points of the Soderberg line are:

To determine whether or not failure will occur on certain planes making an angle  with the horizontal, a point is plotted in the figure below for each value of  for . They fall on a quarter-ellipse.

The safe-stress line is a line tangent to the ellipse and parallel to the Soderberg line. The slope of the Soderberg line is . Therefore, the slope of the safe-stress line is:

.

Solve for . For that value of ,

will give the point G. But solving for  above is difficult.

So, for safe-stress line using the relationship:	 as shown below:

From the proportional triangles GHF and DOF of the figure above:

Where:

We are interested in the stress combination that produces the smallest , since this corresponds to a maximum-stress situation. The minimum value of  corresponds to a maximum value of . Differentiating  and equating the result to zero gives:

. The angle  can now be determined, and the exact value of  at G.

This relationship is illustrated in Figure below:

This shows that:

Substituting these into equation:

Substituting ) into the above equation gives:

This equation above can be compared to:

Biaxial stress situation where , giving the maximum shear stress as:

And this gives a safety factor of:

And the Soderberg line:	, and

which leads back to:

For the case of bending and torsion:

This implies:

Or

These equations for the diameter and safety factor represent the general form of the MSST. Note that in the equation for the diameter that Sy, Se, Kf and Kfs depend on the shaft diameter d. The approach to be used is to estimate the range of the shaft diameter and iterate the results if the final design is outside the estimated range.

Application To DET and Goodman Line

Biaxial stress situation where , giving the principal stresses as:	,

And this gives a safety factor of:

And the Goodman line:	, and

which leads back to:

Given:	When a rear-wheel-drive car accelerates around a bend at high speeds, the drive-shafts are subjected to both bending and torsion. The acceleration torque T is reasonably constant at 400N.m while the bending moment is varying due to cornering and is expressed in newton-meters as

Thus, the mean and alternating moments are  respectively.

Assume there is no notch giving a stress concentration. The reliability must be 99%, and the safety factor is 4.5. The shaft is forged from AISI 1080 steel.

Find:	The shaft diameter while using the MSST.

Solution:	From

and from table, for AISI 1080 (high-carbon)

The surface finish factor for a forged surface at Su = 615 MPa  b1=0.42. In evaluating the size factor, the shaft diameter needs to be chosen.

For bending or torsion:

and for axial:		b2=1

assuming d = 30 mm,