SHAFT-HUB CONNECTION

The varieties and various forms of connection of shafts, axles and steps (tenons) with hubs of gears, rope-pulleys, levers and similar parts can be classified, based the mode of force transfer, into the following:

- Material Joint,

- Positive Joint,

- Pre-stressed Positive Joint and

- Force (Frictional) Joint.

- Material Joint

This type of joints can be achieved by soldering, gluing, or welding. They are termed as permanent joints and they can only be disconnected by destroying them.

- Positive and Pre-stressed Positive Joints

Positive joints transmit force or torque due to their form.

Dynamic forces are better transmitted by pre-stressed positive joints. The contact surfaces of this joint are perpendicular to the forces to be transmitted, and they are kept together by pre-forces.

Positive and pre-stressed positive joints are detachable joints.

Types of Keys

- Sunk keys,	(Passfederverbindung)

- Saddle keys,

- Tangent keys,

- Pin (Round keys), 	(Stiftverbindung)

- Profile of shaft	(Profilwellenverbindung)

Factors affecting selection of Keys

- Power to be transmitted

- Tightness of fit

- Stability of connection, and

- Cost.

Sunk Keys

They are provided half in the keyway of the shaft and half in the keyway of the hub or boss of the pulley.

Advantages:

- Good centring of the keyway in the hub,

- Cheap and easily assembling,

- No kipping

Disadvantages:

- The hub must be axially positioned (locked),

- It is sensitive to dynamic loading due to the clearance between the shaft/hub keyway,

- It weakens the shaft due to the notch-effect,

- Imbalance at high speeds. Therefore, place a second key at 180o.

The sunk-keys are of the following types:

- Rectangular sunk key

taper 	1 in 100 on the top side only.

Width of key,	w = d / 4 and

thickness of key	t = 2w/3 = d/6

where	d = Diameter of the shaft or diameter of the hole in the hub.

- Square sunk key. The only difference between a rectangular sunk key and a square sunk key is that its width and thickness are equal, i.e. w = t = dl4.

- Parallel sunk keys may have rectangular or square section, but uniform in width and thickness throughout is used where the pulley, gear or other mating piece is required to slide along the shaft.

- Gib-head key is a rectangular sunk key with a head at one end known as gib head. It is usually provided to facilitate the removal of key.

Width,	w = d/4

and thickness at large end,	t = 2w/3 = d/6

- Feather key is attached to one member of a pair and permits relative axial movement. It is a special type of parallel key. It is fastened either to the shaft or hub, the key being a sliding fit in the key way of the moving piece.

The feather key may be screwed to the shaft (a) or it may have double gib heads (b).

The following table shows the proportions of standard parallel, tapered and gib head keys, according to IS : 2292 and 2293-1974 (Reaffirmed 1992).

| Shaft diameter (mm) upto and including | Key cross-section |  | Shaft diameter (mm) upto and including | Key cross-section |  |
| --- | --- | --- | --- | --- | --- |
|  | Width (mm) | Thickness (mm) |  | Width (mm) | Thickness (mm) |
| 6 | 2 | 2 | 85 | 25 | 14 |
| 8 | 2 | 3 | 95 | 28 | 16 |
| 10 | 4 | 4 | 110 | 32 | 18 |
| 12 | 5 | 5 | 130 | 36 | 20 |
| 17 | 6 | 6 | 150 | 40 | 22 |
| 22 | 8 | 7 | 170 | 45 | 25 |
| 30 | 10 | 8 | 200 | 50 | 28 |
| 38 | 12 | 8 | 230 | 56 | 32 |
| 44 | 14 | 9 | 260 | 63 | 32 |
| 50 | 16 | 10 | 290 | 70 | 36 |
| 58 | 18 | 11 | 330 | 80 | 40 |
| 65 | 20 | 12 | 380 | 90 | 45 |
| 75 | 22 | 14 | 440 | 100 | 50 |

- Woodruff is an easily adjustable key and capable of tilting in a recess milled out in the shaft by a cutter having the same curvature as the disc from which the key is made. This key is largely used in machine tool and automobile construction.

Saddle keys

- Flat saddle key is a taper key which fits in a keyway in the hub and is flat on the shaft. It is likely to slip round the shaft under load. Therefore it is used for comparatively light loads.

- A hollow saddle key is a taper key which fits in a keyway in the hub and the bottom of the key is shaped to fit the curved surface of the shaft. Since hollow saddle keys hold on by friction, therefore these are suitable for light loads. It is usually used as a temporary fastening in fixing and setting eccentrics, cams etc.

Tangent Keys

The tangent keys are fitted in pair at right angles as shown above. Each key is to withstand torsion in one direction only. These are used in large heavy duty shafts.

Pins

- Round keys are cylindrical section and fit into holes drilled partly in both the shaft and hub. They are most appropriate for low power drives.

- Tapered pins are circular in cross-section and are usually considered to be appropriate for low power drives.

Profile of Shaft

They are made integral with the shaft. They are suitable for the transmission of bigger and dynamic torques due to the uniform distribution off the load on the circumference of the shaft.

Splines

They are suitable for stationary connection as in the case of the driving shaft of an automobile, as well as for axial sliding as with the gears of machine tools.

Serrations

Involute profile Tooth

Polygon Profile

Forces acting on a Sunk Key

The width and height of sunk-keys are standardised according to the diameter of the shaft (DIN 6885). The general procedures of selecting sunk-keys are:

- Load-bearing length of key should not be bigger than the diameter of the shaft, , because an increase in length beyond that will not reduce the pressure. This is due to the fact that the strength key-material is bigger than that of the shaft.

- A stiffer shaft will lead to a uniform distributed force. Therefore, the ratio of the shear modulus of rigidity, G, of shaft/hub must be large.

- The point of load withdrawal (output) must be far away from the point of load introduction ().

- In order to minimise self-rotation the fit between the key and the keyway is h9/P9.

Due to the relative twisting between the shaft and the hub there is micro-slipping at their contact surfaces. Under dynamic loading, this may lead to frictional corrosion (see figure below).

For a key connecting the shaft and hub:

- T = Torque transmitted by the shaft,

- F = Tangential force acting at the circumference of the shaft,

- d = Diameter of shaft,

- 1 = Length of key,

- w = Width of key.

- h = Thickness of key, and

The key may fail due to shearing or compression (surface-pressure).

Shearing of the key, the tangential shearing force acting at the circumference of the shaft,

Tangential shearing force:

Torque transmitted by the shaft:

Compression of the key:

Tangential shearing force:

Torque transmitted by the shaft:

Tangential shearing force:

In dimensioning the parts, there are three parts involved. Therefore, the dimensioning of the key or keyway will depend on:

- properties of materials of the three parts,

- type of loading

In general, a sunk-key connection needs not to be calculated if the normal dimensions are taken from a table. Keys with shorter length () there is the need to calculate the compression (surface-pressure) of the material with the lowest strength, which is normally the hub. Therefore, the surface-pressure of the keyway of the hub must exceed the allowable pressure.

Keyway:

Shaft:

| Type of Connection | Kf |
| --- | --- |
| Sunk-key | 2.4 3.8 |
| Woodruff | 2…….3 |
| Tangent key | 1.8 2 |
| Splines | 2…..3 |
| Tooth with involute profile | 2 |
| Polygon-profile | 1.0 |

Example	Design the rectangular key for a shaft of 5O mm diameter. The compressional and shearing stresses for the key material are 42 MPa and 70 MPa.

Solution:	Given: d=50 mm, , .

From Table, for a shaft of 50 mm diameter, b= 16 mm, h= 10mm.

The length of key is obtained by considering the key in shearing and compression.

Considering shearing of the key

Torsional shearing strength (or torque transmitted) of the shaft,

Considering compression of the key

Example	A 45 mm diameter shaft is made of steel with yield strength of 400 MPa. A parallel key of size 14 mm wide and 9 mm thick made of steel by way of yield strength of 340 MPa is to be used. Find the required length of key, if the shaft is loaded to transmit the maximum permissible torque. Use maximum shear stress theory and assume a factor of safety of 2.

Solution. Given d=45mm, 6,

According to maximum shear stress theory,

Shaft:	Torsional stress and bending stress

Key:	Compression and shear stresses

Maximum torque transmitted by shaft and key:

Steady:

First of all, let us consider the failure of key due to shearing:

Now considering the failure of key due to compression:

Taking the larger of the two values.

Effect of Keyways

A little consideration will show that the keyway cut into the shaft reduces the load carrying capacity of the shaft. This is due to the stress concentration near the corners of the keyway and reduction in the cross-sectional area of the shaft. In other words, the torsional strength of the shaft is reduced.