THREADED FASTENERS

Threading can be grouped according to their application:

- Fastener,

- Set-screws,

- Tensioning screw,

- Measuring screw.

- Lock screw,

- Transmission (power) screw.

A thread is characterised by:

- Thread form (profile),

- Pitch,

- Lead, and

- Direction of turn.

Method of Production

- Forming:	This results in threads with high fatigue strength. The threads are rolled.

- Cutting:	The threads are produces by turning, milling, grinding or whirling.

Terminology

Major (nominal) diameter (d) is the largest diameter of the internal and external screw thread.

Minor (root) diameter (d3) is the smallest diameter of an external or internal thread,

Pitch. (effective) diameter (d2) is the diameter of an imaginary cylinder,

Pitch is the distance from one point on the thread to the corresponding point on the next in the axial direction.

A parameter that can be used instead of pitch is threads per inch.

Lead is the distance between two corresponding points on the same helix. It is advancement made by a screw for one complete rotation.

Crest is the top surface of the thread.

Root is the bottom surface formed by the two adjacent flanks.

Threads series is applied to threads of any size. Each thread series has the same number of threads per inch.

Forms (profile) of Threads

The choice of the form of the thread depends on its application:

- Fastening screw:	V-shaped,

- Transmission screw:	Acme (Trapezoidal shaped). Buttress (Saw-shaped), or round threads.

Commercial threads according to DIN 202 are:

- Metric ISO-Thread (DIN I3)

There are three types:- fine, medium and coarse threads. Designation eg. M12x1.5 for fine.

- Whitworth Pipe Threads (DIN 259),

- Metric Trapezoidal threads (Acme) ( DIN 103),

They can tolerate high axial loads in both direction. They are used for vice spindles or transmission spindles.

- Metric Saw threads (Buttress thread) (DIN 513),

They bear greater load on one side. They are used as screw presses. spindles on lifting jacks,

- Round thread (Knuckle thread) (DIN 405).

They are less sensitive to dirt and wear.

Lead

Through development the helix angle is obtained:

Materials For Bolts and Nuts

The proof load of a bolt is the maximum load that a bolt can withstand without acquiring a permanent set. The proof strength is the limiting value of the stress determined by the proof load and the tensile stress area.

The yield strength and the proof strength have something in common; the yield strength is usually higher because it is based on 0.2% permanent deformation.

The proof strength defines bolt grades or classes that specify the material, heat treatment, and minimum proof strength for the bolt or screw. Below are tables of SAE grade and Metric grade:

| SAE grade | Head marking | Range of crest diameter, (in) | Ultimate tensile strength Rm, (ksi) | Yield strength, Re, (ksi) | Proof strength, Rproof, (ksi) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | 60 | 36 | 33 |
| 2 |  |  | 74 | 57 | 55 |
|  |  |  | 60 | 36 | 33 |
| 4 |  |  | 115 | 100 | 65 |
| 5 |  |  | 120 | 92 | 85 |
|  |  |  | 105 | 81 | 74 |
| 5.2 |  |  | 120 | 92 | 85 |
| 7 |  |  | 133 | 115 | 105 |
| 8 |  |  | 150 | 130 | 120 |
| 8.2 |  |  | 150 | 130 | 120 |

| Metric grade | Head marking | Crest diameter, d3, mm | Ultimate tensile strength, Rm, MPa | Yield strength, Re, MPa | Proof strength, Rp, MPa |
| --- | --- | --- | --- | --- | --- |
| 4.6 |  | M5-M36 | 400 | 240 | 225 |
| 4.8 |  | M1.6-M16 | 420 | 340 | 310 |
| 5.8 |  | M5-M24 | 520 | 415 | 380 |
| 8.8 |  | M17-M36 | 830 | 660 | 600 |
| 9.8 |  | M1.6-M16 | 900 | 720 | 650 |
| 10.9 |  | M16-M36 | 1040 | 940 | 830 |
| 12.9 |  | M1.6-M36 | 1220 | 1100 | 970 |

The proof stress is comparable to allowable stress.

The metric grade is characterised by two numbers separated by a point:

Those designations in brackets are for nuts with restriction or with no load carrying capacity.

For bolt designations (current):

- First number:	is the hundredth digit of the ultimate tensile strength Rm, in N/mm2.

- 2nd Number:	ten times the ratio of yield strength to (ultimate) tensile strength:

Example:	.

For nuts, grades between 4 and 14 are applied. The number refers to  of the proof strength. The right pairing of bolt and nut is achieved if the first number of the bolt is the same as the nut number.

For example, material of nut 10 must be paired with a bolt of 10.9

Force Transformation of a Thread

The tightening or loosening of a thread can be compared to the moving down or up of a wedge on an inclined plane with the force F.

Cases:

- For a flat thread (). as it is shown in the figure below:

Below is a free-body diagram of the wedge (Nut):

These forces acting on the thread can be graphically analysed:

Case a:	Coefficient of friction =0, the resulting force .

In reality. there is friction involved in thread:

Case b:	During tightening, there will be an advancement only when the resultant force  of the external forces () overcome the forces of . The frictional angle between .

Case c:	During loosening, the frictional force is in the opposite direction.

And the following relationship exists:

- For an inclined thread (). as it is shown in the figure below:

In the place of FN, we can put  and therefore the frictional force becomes

Let

Therefore, for an inclined flank:

- Tightening:

- Loosening:

- Frictional torque within threads:

Tightening And Loosening Torques

In addition to friction within the threads to overcome, one must also overcome the friction either between the nut and clamped part or between the head of the bolt and clamped part:

where:	=coefficient of friction between collar of bolt or base of nut and clamped part.

Power and Efficiency of Screw Drive (Mechanism)

Power:

Efficiency is by definition:

Neglecting friction between the nut or bolt head:

With

LOAD ANALYSIS OF BOLTS AND NUTS

- Static Loading

One should differentiate a thread joint that is pre-stressed from one that is not pre-stressed. However. thread-joints are mostly pre-tensioned with an axial force  that prevents the joint from loosening. The bolt is subjected to tension while the clamped parts to compression. The system of bolt-clamped pacts can be compared to a system of springs. This can be graphical represented in form of force-displacement curve.

In equilibrium, the force from the bolt will be acting on the clamped parts. These two graphs can be brought together.

An external force could come into play. Practically, this external force turns to further stretch the bolt and release the clamped parts from compression. The amount of extension of the bolt is equal the amount of compression of-the clamped members. The force (clamped force) that turns to keep the clamped parts together is . The pre-tensioned force should be high enough to prevent clapping of the clamped components. See figure below.

Stiffness (Spring-constant)

The (spring-constant) stiffness of:

Bolt:

Clamped parts:

The dimensionless stiffness parameter:

The maximum force in bolt is:	.

This analysis assumes that the force is applied under the head of' the bolt and under the head of the nut.

Safety Factor

i.	The limiting value of the stress in the bolt, , is the proof strength ().

The introduction of the load factor, s, gives:	 and

The safety factor is not applied to the preload stress.

This implies:

For  ensures that the bolt stress is less than the proof 'strength.

From , the safety factor can be maximized by having zero pre-load on the bolt.

ii.	Where separation of the clamped parts is not concern, the above statement is certainly true. However, a preload is often applied to bolted connections to make certain that members are tightly joined in order to minimise tolerances and provide tight fits. and avoid separation.

Separation occurs when .

is the maximum value of the external force that will cause separation. Thus. the safety factor guarding against joint separation is:

By substituting  implies:

The amount of preload that is in practice applied to bolts under static conditions is therefore a compromise between bolt overloading (where zero preload is most beneficial) and separation.

Where a large preload is desirable, for a safe joint, it is required that the external load be smaller than that load needed to cause the joint to separate. If separation does occur then the entire external load will be imposed on the bolt.

The preload given for re-used and permanent connections are as follows:

Where the proof strength,,for a material is not given, use:

In design. preloads are rarely specified because preloads are extremely difficult to measure during assembly of bolted connections. Two alternatives are used:

i.	Specify the toque which is controlled by using a torque wrench (friction varies). and

ii.	Determine the number of rotations from the snug state such as half-turn. (it is subjective).

The figure below shows the stress-strain diagram of a good quality bolt material. The yield point is clearly not defined and the curve progresses smoothly up to fracture. This means that no matter how much preload is given this bolt, it will retain its load-carrying capacity. This is what keeps the bolt tight and determines the joint strength. Therefore, if the bolt's full strength is not used in developing the pre-tension, then money is wasted and the joint is weaker.

A good-quality bolt can be stressed by external loaded into the plastic range and still maintain tight connections. However. there will be a reduction in the preload. See figure above.

Some of the bolt toque used in tightening produces torsion, which increases the principal tensile stress. However. this torsion is held only by the bolt head and nut; in time it relaxes and lowers the bolt tension slightly. Thus, as a rule, the bolt will either fracture during tightening or not at all.

- Dynamic Loading

A practical situation also arises where the external force is dynamic. Examples of such situations are the rotation of an unbalanced mass, and fluctuation gas pressure in a cylinder. The maximum and minimum external forces .respectively come into play.

Due to the dynamic external load. the bolt will be subjected to a force alternating between maximum and minimum forces:

Or be represented by a mean force:

Superimposed by

Setting And Slacking of Bolted Joints

The surface roughness of load bearing surfaces can be plastically deformed by the preload. This process of levelling of the surf-ace roughness takes place with time and it is term setting.

The setting process causes a reduction of the pre-load by  as a result of reduction in length, . The magnitude of , depends on the number of pairing surfaces (an engaged thread should be counted as pairing surface).

Effect of Stiffness

A less stiff member implies , is small so that x will increase.

Since

The dimensionless stiffness parameter:

This result is valid only as long as clamping load remains in the members.

By applying a less stiff bolt against a high stiff clamped member, the external force (dynamic or Static) component of the holt will be small.

Stiffness Determination of Bolt and Clamped Parts

Tightening of a thread-joint results in stretching of the bolt. On the other hand the clamped parts are compressed.

Elasticity of a body is the amount by which the body will stretch or compress when subjected to a unit force, eg 1N.

Stiffness of a Bolt

In order to calculate the elasticity of a bolt, the various sections (varied diameters and length) of the bolt that are serially connected must be taken into consideration:

- ,

The effective length of the threaded portion extends into the nut, while the shank extends into the head of the bolt. For standard heads and nuts the following effective lengths must be taken into consideration:

m=thickness of nut

| Bolt Head |  |
| --- | --- |
| Hexagonal head |  |
| Internal hexagonal head |  |
| Nut |  |
| m/d=0.8 |  |
| m/d=1.25 |  |
| m/d=1.5 |  |

Stiffness of Clamped Parts

The determination of the elasticity of the clamped parts,  is much complicated, because:

- Consists of parts with different properties (eg E).

- The exact cross-sectional area (portion of material involved in the compression) is not known.

Determination of :

- Thin sleeve:

For clamped members that tend to experience transverse compression form conical pressure members:

- steel

is diameter of washer or the distance across-flats of the head of bolt or nut.

Force Introduction in Bolt Connection

Under normal operating conditions the operating force  (or ) does not unlike  act under the head of the bolt or nut. It acts within the clamped length . The region between the points of action therefore forms the clamped length. The region(s) outside these points must now be serially connected to the bolt. Therefore. the more these points move inward the less stiff the bolt becomes. On the other hand, the clamped members become stiffer.

On the basis of  the stiffness of bolt and clamped members remain unchanged ( and ), because this force really acts underneath the head of the bolt and nut. The dynamic loading of the bolt therefore becomes smaller. This is an advantage.

Where the points of action are not known n=1 will be on a safer side.

In the case the force  acts at the joint interface (n=0), , degenerates into a vertical line (that is no external load on bolt). However, when  clapping occurs.

THREAD STRENGTH (Stresses In The Tooth)

In order to ascertain the strength of a bolt-,joint, the allowable stress must he compressed to an equivalent stress determined by a failure criterion.

When the DET (distortion energy theory) is applied:

- Bending stress, , at the crest-diameter ():

Canter lever

For Metric ISO V-thread

d

- Compressional stress

From the radial component,

- Transverse shear stress

Under the assumption that there is a uniform distribution of load on the number of teeth engaged at .

If the allowable stress is equated to the tensile strength of the bolt, the critical nut-thickness or engaged thread length:

The thickness of the nut must be dimensioned such that the full carrying force of the bolt without damage to the tooth of the nut. Care must be taken that the surface-pressure in the engaged teeth be less than the allowable surface-pressure.

For transmission threads, the surface-pressure is also a determining quantity from which the thickness of the nut can be determined.

Design And Strength Calculations Of A Threaded Connection

The determination of the strength of a thread connection requires the static and dynamic loads be separated. By superposition of the dynamic and static stresses the equivalent maximum stress is determined:

1.	Static Loading

It should be noted that when considering the static loading that during the tightening stage the bolt is subject to torsion, so that an equivalent stress is required:

According, to DET von Mises:

- Mean tensile stress:

With the mean force:	  n is not included.

And

- Mean torsional stress:

Maximum constant torque:

And

2.	Dynamic Loading

For loading due solely to dynamic forces:	.

n is not considered because we are interested in the bolt only.

_	The fatigue strength, , can be obtained from tables and diagrams (as shown below) based on the grade.

|  |  | Fatigue strength (N/mm2) Regular threads For threads diameters: |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | <8 | 8-12 | 14-20 | >20 |
| Strength Class | 4.5 and 5.6 | 50 | 40 | 35 | 35 |
|  | 8.8 to 12.9 | 60 | 50 | 40 | 35 |
|  | 10.9 and 12.9 rolled | 100 | 90 | 70 | 60 |

Fig. a :	Fatigue strength of bolts whose threads are tempered and paired with normal nuts.

Fig. b.:	Fatigue Strength of bolts whose threads after tempering they are rolled and paired with normal nuts.

The allowable value for static and dynamic stresses can be obtained from the modified Goodman (Smith) Diagram. For bolts,  is approximately independent from the static stress.

3.	Surface-Pressure

The surface-pressure between nut / bolt-head and the clamped members must he compared to the allowable surface-pressure:

| Material of bearing surface |  | Allowable surface pressure,(N/mm2) |
| --- | --- | --- |
| Steel | St37.C15N | 300 |
|  | St50.C35N | 500 |
|  | St52.C45N | 600 |
|  | Tempered steel Rm>900N/mm2 | 900 |
|  | Tempered steel Rm>1200N/mm2 | 1200 |
|  | C15 surface-hardened to 0.6mm | 1400 |
|  | 16MnCr5 and others surface hardened to 1mm | 1800 |
| Cast-Iron | GG25 | 850 |
|  | GGG50.GTS45 | 500 |
| Al-alloy | GK |  |
|  | MgAl9 | 150 |
|  | GD |  |
|  | GK |  |
|  | AlSi2Cu | 300 |
|  | GD |  |
|  | GK |  |
|  | AlSi6Cu | 250 |
|  | GD |  |

Table:	Allowable surface-pressure of material of bolt-head and nut.

Forming of Thread Connections

There are three basic forms of normal machine-screws:	bolt and nut, cap screw and stub.

- The bolt and nut requires accessibility at both ends such as when applied on flanges, couplings, and pipe-construction. In machine-construction, where accessibility is rare the cap-screw and the stud are frequently encountered.

The fatigue strength of a thread is increased by pairing a thread with an increased longitudinal elasticity:	 and stiffer clamped parts. This put the bolt and nut over the other two clamped members.

- The high-rigid hexagonal socket head-bolt reduces weight and space.

- In highly stressed construction such as the crank-shaft. the application of elastic-bolt and tensile-nut is appropriate.

iv.	In thread-connection with short screws the transition from the shank to the head of the bolt as well as the head itself can be elastically formed in order to minimise the magnitude of the alternating stress.

v.	Fatigue strength of a thread-connection can be almost doubled by rolling, and tempering the threads. This increase in strength can be attained only if the notch-effect at the transition to the bolt-head as well as at the first thread are avoided.

vi.	One possibility of raising the capacity of' the threads of the nut made from a low strength material is the application of threaded-bushing.

Transverse Loading of a Thread Connection

Tight-fit screw (shank fit H7-k6), shear bushing and sleeve are subjected to shear and bearing surface pressure:

Recommended values for general machine construction are:	Rp0,2= (yield strength) of the screw or bushing. and Rm (tensile strength) of the weakest pairing material respectively.

a	Static

b	Released tension

c	Completely reversible

Securing Threads

A properly mounted. properly constructively designed thread connection that has its pre-load during operation not changing can fall out if it is not secured. All measures to increase the clamping force are the same measures taken to raise the safety of the connection.

The failure of a thread connection is associated with the lost of the clamping force. One should differentiate the following:

Slacking:	is the lost of the clamping force as a result of setting, and

Loose-turning:	is a result of sliding moment.

Thread connection securing elements are therefore grouped under slack securing elements and loose-turning securing elements.

Self-Locking Screws