INTRODUCTION

The purpose of a bearing is to provide relative positioning and rotational freedom while transmitting a load between two structures, usually a shaft and housing.

BEARING TYPES

Ball and roller bearings are available to the designer in a great variety of designs and size ranges.

Rolling-element bearings are an assembly of several parts:

- An inner race (ring),

- Outer race (ring),

- A set of balls or rollers, and

- A cage or retainer (separator)

The cage maintains even spacing of the rolling element. A cage-less bearing, in which the annulus is packed with the maximum rolling-element, is called a full-complement bearing. Full-complement bearings have high load-carrying capacity but lower speed limits than bearings equipped with cages. Although rolling-element bearings will function well without a lubricant, it is often advantageous to provide a lubricant film to extend their life.

- BALL BEARING

They are used in greater quantity than any other type of rolling-element bearing. For an application where the load is primarily radial with some thrust load present, one of the radial ball bearings cited below can be chosen.

- Radial Ball Bearing

- Deep-Groove Bearing:	It has a ball complement limited by the number of balls that can be packed into the annulus between the inner and outer races with the inner race resting against the inside diameter of the outer race.

- Filled Notch Bearing:	A filled notch bearing most often has both inner and outer races notched so that a ball complement limited only by the annular space between the races can be used. It has low thrust capacity.

- Internal Self-Aligning Bearing:	It has an outer-race ball path ground in a spherical shape so that it can accept high levels of misalignment.

- External Self-Aligning Bearing:	It has a multi-piece outer race with a spherical inner race. It can accept high misalignment and has higher capacity than the self-aligning internal bearing.

- Double row

Angular-Contact Ball Bearings

An angular contact ball bearing has a two-shouldered ball groove in one race and a single-shouldered in the other race. Thus, it can support only a unidirectional thrust load.

They can be mounted as duplex pairs either back-to-back or face-face as shown above. Duplex bearing pairs are manufactured so that they preload each other when clamped together in the housing and on the shaft.

Preloading provides stiffer shaft support and helps prevent bearing skidding at light loads. Proper levels of preload can be obtained from the manufacturer. A duplex pair can support bidirectional thrust load. The back-to-back arrangement offers greater resistance to moment or overturning loads than does the face-face arrangement.

Where thrust loads exceed the load-carrying capacity of a simple bearing, two bearings can be used in tandem, with each bearing supporting part of the thrust load. Three or more bearings are occasionally used in tandem, but this is discouraged because of the difficulty in achieving effective load sharing:	even slight differences in operating temperature can cause an uneven distribution of load sharing.

- Thrust Ball Bearing

It is used exclusively for machinery with vertically oriented shafts.

Flat-race Bearing:	allows eccentricity of the fixed and rotating members:	an additional bearing must be used for radial positioning. It has a low thrust load-carrying capacity because of its small ball-race contact and subsequent high Hertzian stress.

Grooved-race Bearings :	have higher thrust load-carrying capacity and can support low-magnitude radial loads.

- ROLLER BEARINGS

- Cylindrical Roller Bearings

They provide purely radial load support in most applications.

An N or NU-type bearing will allow free axial movement of the shaft relative to the housing to accommodate differences in thermal growth.

An F- or J-type bearing will support a light thrust load in one direction and a T-type bearing, a light bidirectional thrust

Cylindrical roller bearings have moderately high radial load-carrying capacity as well as high-speed capacity, exceeding those of either spherical or tapered-roller bearings.

Separable outer ring, non-locating (N)

Separable inner ring, non-locating (NU)

Separable inner ring, one-direction locating (NJ)

- Spherical Roller Bearings

The more popular bearing design (convex) uses barrel-shaped rollers. An alternative design (concave) employs hourglass-shaped rollers.

Que:	The geometry of the rolling surfaces in rolling-element bearings is determined from the load carrying capacity of the elements and the type of motion. The load in ball bearings is carried by the balls. Depending on the geometry of the races, radial, axial, or moment loads can transmitted through a ball bearing.

Determine which ball bearing type should be used for:

- Pure radial load,

- Pure axial load,

- Bending moment load,

- High radial stiffness and

- High axial stiffness.

Solution:

- For pure radial loading, a radial ball bearing is recommended. Conrad (deep-groove) ball bearings are the least recommended. A filled-notch bearing offers additional radial capacity. If there are no space limitations and the bearing width is not restricted, a double-row bearing should be considered.

- For pure axial loading the force transmitted through the balls should be as close as possible to the bearing axis. This location gives the smallest possible internal force acting through the balls.

- For bending moment loading a bearing must support loads in both the axial and radial directions; deep-groove and double deep-groove ball bearings can be used. If two bearings are mounted so far apart that the center distance between the balls is about the same as the bearing outer diameter.

- The elastic deformation in a ball bearing is related to the load W as follows:

and the contact stiffness is:

When W approaches zero, the contact stiffness also approaches zero; thus the only way to get high radial stiffness in a ball bearing is to preload the balls. To obtain high radial and high axial stiffness and high running accuracy, pairs of angular-contact bearings are used with axial preloading against each other. The same effect can be achieved by using a single bearing with a split ring.

- High axial stiffness can be achieved by preloading thrust ball bearings, such as deep- groove or angular-contact ball bearings, against each other. The higher the preload, the higher the stiffness becomes, but at the same time the shorter the bearing life becomes and the higher the power loss (temperature) becomes.

GEOMETRY

The operating characteristics of a rolling-element bearing depend greatly on it diametral clearance ().

Ball Bearing

- Pitch diameter  is the mean of the inner and outer-race contact diameters and is give by:

- Diametral clearance :	is the maximum distance that one race can move diametrally with respect to the other when non-measurable force is applied and both races lie in the same plane.

- Race conformity:	 is a measure of the geometric conformity of the race and the ball in a plane passing through the bearing axis. Thus, inner race conformity and outer race conformity.

Race conformity can be expressed as:

In perfect conformity, the race radius is equal to the ball radius: thus . The closer the race conforms to the ball, the greater the frictional heat within the contact. On the other hand, open-race curvature and reduced geometric conformity, which reduces friction but increases the maximum contact stresses and consequently reduce the bearing fatigue life.

The race conformity ratio for the outer race is usually made slightly larger than that for the inner race.

STATIC LOAD DISTRIDUTION

Most rolling-element bearing applications involve steady-state rotation of the inner race or outer race or both. However, the rotational speeds are usually not so high as to cause ball or roller centrifugal forces or gyroscopic moments of significant magnitude. In analyzing the loading distribution on the rolling elements, it is usually satisfactory to ignore these effects in most applications.

Radially Loaded Ball and Roller Bearings

In concentric arrangement, radially loaded rolling elements will have a uniform radial clearance of . The application of an arbitrarily small radial load to the shaft will cause the inner race to move a distance  before contact is made between a rolling element located on the load line and the inner and outer races.

Thrust-Loaded Ball Bearings

The static thrust load-carrying capacity of a ball bearing may be defined as the maximum thrust load that the bearing can endure before the contact ellipse approaches a race shoulder as shown below:

Or as the load at which the allowable mean compressive stress is reached, whichever is smaller. Both the limiting shoulder height and the mean compressive must be calculated to find the static thrust load-carrying capacity.

Preloading

The object of preloading is to:

- eliminate the internal clearance found in bearings,

- increase the fatigue life, and

- decrease the shaft slope at the bearing.

Preloading of straight roller bearings may be obtained by:

- Mounting the bearing on a tapered shaft or sleeve in order to expand the inner ring,

- Using an interference fit for the outer ring

Angular-contact bearings as duplex pairs are usually preloaded against each other. Duplex bearing pairs are used in either back-to-back or face-to-face arrangements. Such bearings are usually preloaded against each other by providing what is called stick-out in the manufacture of the bearing. The figure illustrates this for a bearing pair used in a back-to-back arrangement.

The magnitude of the stick-out and the bearing design determine the level of preload on each bearing when the bearings are clamped together.

The figure below shows the relationship of initial preload, system load, and final load for bearings a and b.

The load deflection curve follows the relationship. When a system thrust load Pt, is imposed on the bearing pairs, the magnitude of load on bearing b increases while that on bearing a decreases until the difference equals the system load. The physical situation demands that the change in each bearing deflection be the same.

The increments in bearing load, however, are not the same. This is important because it always requires a system thrust load far greater than twice the preload before bearing becomes unloaded. Prevention of bearing unloading, which can result in skidding and early failure, is an objective of preloading.

STATIC LOAD RATING (CAPACITY)

For still-stand or very low operating speeds of bearing.

The static load rating for rolling-element bearings represents the loads at which, under defined operating conditions, the load carried by the most heavily loaded rolling element  equals the allowable rolling-element load , which is the maximum load before plastic (rather than elastic) deformation occurs within the bearing.

The static load capacity is the load, which at the highest stressed location between the rolling elements and the rings that causes a plastic deformation of 0.0001 times the diameter of the rolling element.

The static load rating  can be expressed as:

Static Thrust Load-Carrying Capacity

It is the load that gives small plastic deformation of the races.

Allowable load limit may be viewed as the maximum bearing load where fatigue failure will not occur. Thus, if the design load were equivalent to the allowable load limit, the safety factor would be 1 for infinite life.

The speed ratings in the table correspond to the maximum speed the bearing should experience before the bearing design must be altered to handle the high temperature developed in the bearing.

Equivalent Static Load

A load P acting on a rolling-element bearing at an angle, , is a combined load, since it contains both a radial and an axial component.

The radial component is:	 and

The axial component is:

And the Equivalent static load is:

where X0 = radial factor and YO = thrust factor.

The table is for statically stressed radial bearings

| Bearing type |  | Single Row |  | Double Row |  |
| --- | --- | --- | --- | --- | --- |
|  |  | X0 | Y0 | X0 | Y0 |
| Radial deep-groove ball |  | 0.6 | 0.5 | 0.6 | 0.5 |
| Radial angular-contact ball |  | 0.5 | 0.42 | 1 | 0.84 |
|  |  | 0.5 | 0.38 | 1 | 0.76 |
|  |  | 0.5 | 0.33 | 1 | 0.66 |
|  |  | 0.5 | 0.29 | 1 | 0.58 |
|  |  | 0.5 | 0.26 | 1 | 0.52 |
| Radial self-aligning ball |  | 0.5 | 0.22cot  | 1 | 0.44cot  |
| Radial spherical roller |  | 0.5 | 0.22cot  | 1 | 0.44cot  |
| Radial tapered roller |  | 0.5 | 0.22cot  | 1 | 0.44cot  |

Given:	A single-row, angular-contact ball bearing loaded with a static load has an axial component of 1000 N and a radial component of 500 N. The static basic load rating is 1500 N.

Find:	The contact angle, β, between 20° and 40° that produces the largest static safety factor.

Solution:	For a single row radial angular-contact bearing:

The safety factor is the largest when PO is the smallest, which means that when Yo is the smallest. This implies that the contact angle should be 40o, thus producing Yo = 0.26.

. .	PO = 250 + (0.26)(1000) = 510 N

FATIGUE-LIFF

If a bearing is clean and properly lubricated, mounted and sealed against dust or dirt and operated at reasonable temperatures, then material fatigue will be the only cause of failure.

Contact Fatigue Theory

Rolling fatigue is a material failure caused by the application of repeated stresses to a small volume of material. On a microscopic scale there will be a wide dispersion in material strength or resistance to fatigue because of inhomogeneities in the material.

Because bearing materials are complex alloys, they are not homogeneous or equally resistant to failure at all points. Therefore, the fatigue process can be expected to be one in which a group of seemingly identical specimens exhibit wide variations in failure time when stressed in the same way. For this reason it is necessary to treat the fatigue process statistically.

Predicting how long a particular bearing will run under a specific load requires the following essential pieces of information:

1.	An accurate, quantitative estimate of life dispersion or scatter

2.	The life at a given survival rate or reliability level

This translates to an expression for the load-carrying capacity, or the ability of the bearing to endure a given load for a stipulated number of stress cycles or revolution. If a group of supposedly identical bearings is tested at a specific load and speed, the distribution in bearing life is shown below:

Life (fatigue life):	of an individual bearing is the total number of revolutions or hours attained at a given constant speed of bearing operation for fatigue to be evident on one of its rings or rolling elements.

Useful Life may extend considerable beyond the fatigue life.

Nominal or Rating life 	of a group of apparently identical ball or roller bearings is defined as the number of revolutions or hours at some constant speed, that 90% of a group of bearings will complete or exceed before the failure criterion develops.

The term Minimum life (L10)	is also used to denote rating life.

The term average life or median life are both used in discussing the longevity of bearings

Weibull Distribution

Weibull (1949) postulated that the fatigue life of a homogeneous group of rolling-element bearings are dispersed according to:

lnln1S=e1lnLA

Where:	S is the probability of survival

L is the fatigue life, and

e1 and A are constants

The Weibull distribution results from a statistical theory of strength based on probability theory, where the dependence of strength on volume is explained by the dispersion strength. This is the "weakest link" theory.

The Weibull distribution has two principal functions:

- Bearing fatigue life’s plot as a straight line on Weibull coordinates log-logversus log, so that the life at any reliability level can be determined.

Of greatest interest are the L10 life S=0.9 and the L50 life S=0.5. Bearing load ratings are based on the L10 life.

- Equation lnln1S=e1lnLA can be used to determine the L10 life or to obtain a required life at any reliability level.

The L10 life is calculated from the load on the bearing and the bearing dynamic capacity, or by the load rating given in manufacturers’ catalogs and engineering journals, by using the equation:

.

where :	C = specific dynamic capacity or basic dynamic load rating, N

P = equivalent bearing load, N

mk= load-life exponent:	3 for elliptic contacts and 10/3 for rectangular contacts.

The bearing life in hours is:

Lh=106L60Nb

Where	Nb = rotational speed (rpm).

Dynamic Load Rating

Bearing ratings are based on Lundberg-Palmgren theory.

Two groups of identical bearings tested under different loads F1 and F2 will have respective lives L1 and L2 according to the relation:

L1L2=F2F1mk

A standard of reference is established for bearing load ratings in which speed is not considered.

This bearing load rating is expressed in terms of the dynamic load rating, which, by definition, is the load that a bearing can carry for 1 million inner-race revolutions with a 90% chance of survival.

Dynamic capacity:	 is the constant load that a group of apparently identical bearings can endure for a nominal life of 106 revolutions at constant speed and rotating load for inner ring and stationary load for outer ring.

Factors on which specific dynamic capacity and bearing life depend are:

- Size of rolling element,

- Number of rolling elements per row,

- Number of rows of rolling elements,

- Conformity between rolling elements and races,

- Contact angle under load,

- Material properties,

- Lubricant properties,

- Operational temperature,

- Operating speed.

Only factors i to v are incorporated in bearing dynamic load ratings developed from the Lundberg-Palmgren theory (1947. I952).

The table below gives the dynamic load ratings for single-row deep-groove ball bearings:

|  |  |  | Single-Row Bearing |  |  |  | Double-Row Bearing |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PaPr≤e |  | PaPr>e |  | PaPr≤e |  | PaPr>e |  |
| Bearing Type |  | e | X | Y | X | Y | X | Y | X | Y |
| Deep-groove ball bearings | Pa/C0=0.025 | 0.22 | 1 | 0 | 0.56 | 2.0 |  |  |  |  |
|  | Pa/C0=0.04 | 0.24 | 1 | 0 | 0.56 | 1.8 |  |  |  |  |
|  | Pa/C0=0.07 | 0.27 | 1 | 0 | 0.56 | 1.6 |  |  |  |  |
|  | Pa/C0=0.13 | 0.31 | 1 | 0 | 0.56 | 1.4 |  |  |  |  |
|  | Pa/C0=0.25 | 0.37 | 1 | 0 | 0.56 | 1.2 |  |  |  |  |
|  | Pa/C0=0.50 | 0.44 | 1 | 0 | 0.56 | 1 |  |  |  |  |
| Angular-contact ball bearing |  | 0.57 | 1 | 0 | 0.43 | 1 | 1 | 1.09 | 0.70 | 1.63 |
|  |  | 0.68 | 1 | 0 | 0.41 | 0.87 | 1 | 0.92 | 0.67 | 1.41 |
|  |  | 0.80 | 1 | 0 | 0.39 | 0.76 | 1 | 0.78 | 0.63 | 1.24 |
|  |  | 0.95 | 1 | 0 | 0.37 | 0.66 | 1 | 0.66 | 0.60 | 1.07 |
|  |  | 1.14 | 1 | 0 | 0.35 | 0.57 | 1 | 0.55 | 0.57 | 0.93 |
|  |  | 1.33 | 1 | 0 | 0.33 | 0.50 | 1 | 0.47 | 0.54 | 0.81 |
| Self-aligning ball bearing |  | 1.5tan |  |  |  |  | 1 | 0.42cot | 0.65 | 0.65cot |
| Spherical roller bearing |  | 1.5tan |  |  |  |  | 1 | 0.45cot | 0.67 | 0.67cot |
| Tapered roller bearing |  | 1.5tan | 1 | 0 | 0.40 | 0.4cot | 1 | 0.42cot | 0.67 | 0.67cot |

And the table below is for single-row, cylindrical roller bearings.

| BEARING TYPE |  |  | Single-Acting |  | Double-Acting |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PaPr>e |  | PaPr≤e |  | PaPr>e |  |
|  |  | e | X | Y | X | Y | X | Y |
| Thrust bearing |  | 1.25 | 0.66 | 1 | 1.18 | 0.9 | 0.66 | 1 |
|  |  | 2.17 | 0.92 | 1 | 1.90 | 0.55 | 0.92 | 1 |
|  |  | 4.67 | 1.66 | 1 | 3.89 | 0.52 | 1.66 | 1 |
| Spherical roller thrust |  | 1.5tan | tan  | 1 | 1.5tan | 0.67 | tan  | 1 |
| Tapered roller |  | 1.5tan | tan  | 1 | 1.5tan | 0.67 | tan  | 1 |

The dynamic load ratings are based on the material and manufacturing techniques used for producing standard production bearings. They apply to loads that are constant in both magnitude and direction. For:

- Radial bearings these loads are radial, and

- Thrust bearings they are axial loads that act centrically.

Equivalent Dynamic Load

Equations used for calculating the fatigue life, assumes a purely radial load in radial bearings and purely axial load in thrust bearings, both loads of constant direction and magnitude.

In many rolling-element bearing applications, the load does not satisfy these conditions, since the force acts on the bearing obliquely or changes in magnitude. In such cases, a constant radial or axial force must be determined for the rating life calculation, representing, with respect to the rating life, an equivalent stress.

Axial bearings take only axial load, and radial bearings take both radial and axial loads.

We can also have a rotating load on the outer ring and a stationary load on the inner ring.

- Constant speed, radial and axial loads :

- Rotating load for inner ring / stationary load for outer ring, or

- Stationary load for inner ring / rotating load for outer ring.

The equivalent load is expressed as:

Where:	X=radial factor

Y=axial factor,

Pr=radial load component, N

Pa=axial load component, N

V=rotation factor	(1 for rotating inner ring and 1.2 for rotating outer ring)

Also CO, is the static load rating.

- Varying Speed, Radial and Axial Loads

We would not consider this case.

Life Adjustment Factors

Factors affecting the fatigue life of bearings that were not taken into account in the Lundberg-Palmgren theory are considered here. It is assumed that the various environmental or bearing design factors are multiplicative in their effects on bearing life. The following equations result:

LA=DEFtGHmL10

LA=DEFtGHmCPmk

Where:	D=material factor

E=metallurgical processing factor

Ft=lubrication factor

G=speed effect factor

Hm=misalignment factor

P=bearing equivalent load, N

mk=load-life exponent: 3 for ball bearings or 10/3 for roller bearing

C=specific dynamic capacity, N

BEARING MOUNTING

In a shaft with a system of bearings there should be only one bearing designed to take axial load.

- Fixed and loose bearing

- Stutz-traglagerung

- X-anordnung