The figure below shows a sectional view of a spur-gear transmission system consisting of a gear-box and a motor that have been built together. Within the gear-box there are four spur gears (1, 2, 3 and 4) mounted on three shafts. Gear 1 is mounted on the shaft of the motor and gear 3 is mounted on the output shaft.

Bearings A and B, and gears 2 and 4 are mounted on the transitional shaft.

The top right figure shows the side view of gears 2 and 4 indicating the direction of the forces acting on them. The data below is provided.

Data:

Power:	4kW

Rotational speed of transitional shaft

Pitch diameter of gear 1:	40mm

Pitch diameter of gear 2:	160mm

Pitch diameter of gear 3:	50mm

Pitch diameter of gear 4:	150mm

Material: St 50

| ultimate | Yield | Hardness | Fatigue limit or fatigue strength |  |  |  |  |  | Modulus |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | bS | HB | zSch | zW | bSch | bW | tSch | tW | E- | G- |
|  | zS |  |  |  |  |  |  |  |  |  |
|  | tS |  |  |  |  |  |  |  |  |  |
| 500 to 600 | 450 | 1400 to 1700 | 320 | 220 | 400 | 250 | 180 | 150 | 210000 | 80000 |
|  | 320 |  |  |  |  |  |  |  |  |  |
|  | 180 |  |  |  |  |  |  |  |  |  |

The table below gives the size factor b1 coefficient for combined bending and torsion:

| d mm) | 10 | 15 | 20 | 30 | 40 | 60 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b1 | 1.0 | 0.98 | 0.95 | 0.90 | 0.85 | 0.80 | 0.75 |

The table below gives b2 coefficient for surface finish:

| B | 300 | 400 | 500 | 600 | 700 | 800 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Polished | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Fine turning | 0.97 | 0.96 | 0.95 | 0.94 | 0.935 | 0.932 | 0.93 |

Que. 1:	Assuming there are no losses involved in the transmission, determine the forces

Que. 2:	Determine the supporting forces acting on the transitional shaft on the respective planes. Show this in a drawing form.

Que. 3:	Provide a graph of the shear forces acting on the shaft in the respective planes.

Que. 4:	Provide a graph of the longitudinal forces acting on the shaft in the respective planes.

Que. 5:	Provide a graph of the bending moment load on the shaft in the respective planes

Que. 6:	Provide a graph of the torsional load on the shaft in the respective planes.

For the cut-section at x-x shown in the detail drawing above:

Que. 7:	Determine the nominal stress values at that section.

Que. 8:	Determine the effective stress values at that section.

Que. 9:	Determine the fatigue strength values.

Que. 10:	Determine b1 and b2.

Que. 11:	Provide the modified Goodman graph for that section.

Que. 12:	Indicate the stress state in the above modified Goodman graph.

Que. 13:	If failure can be as a result of both the mean ad alternating stresses, indicate the load-line in the above graph.

Que. 14:	Determine the safety factor.

The figure below shows two plates that have been clamped together by a bolt. The tightening force is 16kN. An external force (FA) of 8kN acts on the members as indicated in the diagram

Que. 15:	Provide a sketched force-displacement diagram.

Que. 16:	Indicate the resultant force that keeps the plate together.

Que. 17:	Where can we place an elastic member (at A, B or C) so that the clamping force that keeps the plate together will reduce. Give reason(s).

Que. 18:	Where can we place an elastic members (at A, B, or C) so that the resulting force in the bolt will be reduced. Give reasons.

Que. 1

Que. 2

Sign conversion:

For section X-X:

Que. 7:	Nominal stresses:

Transverse shear:

Longitudinal tensile:

Bending tensile:

Torsional shear:

Equivalent stresses:	DET

Que. 8:	Effective stresses:

Que. 9:	Fatigue strength values:

Que. 10:	Size factor:

Que. 15

Que. 17:	A and C

Que. 18:	A and C