The figure below shows the views (Front and Left) of a transitional shaft mounted on two bearings A and B. Two spur-gears (2 and 4) mounted on this shaft are subjected to forces F2 and F4 respectively

The left view shows gears 2 and 4 as well the direction of the forces acting on them. The data below is provided.

Data:

Power:	4kW

Rotational speed of transitional shaft

Effective diameter of gear 2:	160mm

Effective diameter of gear 4:	150mm

Material for the shaft:	St 50

| ultimate | Yield | Hardness | Fatigue limit or fatigue strength |  |  |  |  |  | Modulus |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | bS | HB | zSch | zW | bSch | bW | tSch | tW | E- | G- |
|  | zS |  |  |  |  |  |  |  |  |  |
|  | tS |  |  |  |  |  |  |  |  |  |
| 500 to 600 | 450 | 1400 to 1700 | 320 | 220 | 400 | 250 | 180 | 150 | 210000 | 80000 |
|  | 320 |  |  |  |  |  |  |  |  |  |
|  | 180 |  |  |  |  |  |  |  |  |  |

The table below gives the size factor b1 coefficient for combined bending and torsion:

| d mm) | 10 | 15 | 20 | 30 | 40 | 60 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b1 | 1.0 | 0.98 | 0.95 | 0.90 | 0.85 | 0.80 | 0.75 |

The table below gives b2 coefficient for surface finish:

| B | 300 | 400 | 500 | 600 | 700 | 800 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Polished | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Fine turning | 0.97 | 0.96 | 0.95 | 0.94 | 0.935 | 0.932 | 0.93 |

Que. 1:	Assuming there are no losses involved in the transmission, determine the torque transmitted through the transitional shaft.

Que. 2:	Determine the forces  acting on gears 2 and 4.

Que. 3:	Determine the reacting forces acting on the transitional shaft at bearing A and B (on the respective planes). Show this in a drawing form.

Que. 4:	Provide a graph of the shear forces acting on the shaft in the respective planes.

Que. 5:	Provide a graph of the longitudinal forces acting on the shaft in the respective planes.

Que. 6:	Provide a graph of the bending moment load on the shaft in the respective planes

Que. 7:	Provide a graph of the torsional load on the shaft in the respective planes.

A detail view of Z is also shown above. The profile of the bearing and the cut-section at x-x are captured in this detail section:

Que. 8:	Determine the nominal stress values at that section.

Transverse shear:

Longitudinal tensile:

Bending tensile:

Torsional shear

Que. 9:	Applying DET, determine the equivalent stresses (static and dynamic)

Que. 10:	Determine the effective stress values at that section.

Que. 11:	Determine the fatigue strength values.

Que. 12:	Determine b1 and b2.

Que. 13:	Provide the modified Goodman graph for that section.

Que. 14:	Indicate the stress state in the above modified Goodman graph.

Que. 15:	If failure can be as a result of both the mean ad alternating stresses, indicate the load-line in the above graph.

Que. 16:	Determine the safety factor.

Que. 1

Que. 2

Sign conversion:

For section X-X:

Que. 7:	Nominal stresses:

Transverse shear:

Longitudinal tensile:

Bending tensile:

Torsional shear:

Equivalent stresses:	DET

Que. 8:	Effective stresses:

Que. 9:	Fatigue strength values:

Que. 10: