# KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY, KUMASI

COLLEGE OF ENGINEERING

FACULTY MECHANICAL AND AGRICULTURAL ENGINEERING

# B.Sc. (Engineering) First Semester Examination, 2008

Third Year

# AE 373 	MACHINE DESIGN 1

November, 2008.		 THREE HOURS

Attempt: All questions in SECTION A, and Two alternatives from SECTION B

CONSTANTS:

SECTION A	30 marks

Que. 1	Appliance (device) is a technical system whose primary objective is primarily to cause the flow and transformation of:

- mass.

- energy.

- material.

- signal.

- forces

Que. 2	Which of the following combination of technical systems can be classified as appliance?

- Sieve and Turbine,

- Heat pump and Electric shaver,

- Sieve and electric shaver

- Turbine and heat pump

- Television and typing machine,

Que. 3	The following statement(s) beneath is / are true about prescriptive model:

  - Has rules that tend to deliver almost all possible solutions,

  - It is solution-focused,

  - Offers a non-algorithmic procedure, and

  - It is heuristic in nature.

- Only (i),

- Only (ii)

- (i) and (ii),

- (ii) and (iii),

- Only (iv).

Que. 4	The output of the “Analysis of the Problem” consists of one or more of the following basic element(s)

- A statement of the design problem proper,

- Limitations placed upon the solution,

- The criteria of excellence to be worked to

  - Only (i) and (ii),

  - Only (ii) and (iii)

  - Only (i) and (iii)

  - All

  - Only (i)

Que. 5	As an engineer of a company, you have been approached by a client requesting for a system to enable him mill corn at a capacity of 500kg/hr that makes use of electrical power. A general formulation of the “Description of the aim” will be “To design a ”

- Machine to mill corn,

- Technical system to mill corn,

- Technical system to mill corn at a capacity of 500kg/hr,

- Technical system to mill corn at a capacity of 500kg/hr and it must make use of electrical power.

- None of the above is suitable.

Que. 6	The basic structure(s) (based upon the arrangements of the various elementary functional structures) is (are):

- Chain structure,

- Partial structure,

- Parallel structure,

- Cyclic structure

- i, and ii only

- iii and iv only

- i, ii, and iii only

- ii, iii, and iv only

- iii, iv and i only

Que. 7	Which of the load(s) listed below cannot be transmitted by an axle?

- Longitudinal loads,

- Transverse loads,

- Torsional loads,

- Bending moment

  - i,

  - ii,

  - iii,

  - iv,

  - i and ii only

Que. 8:	When can an axle be said to be statically loaded?

    - A rotating axle subjected to a steady and stationary load,

    - A non-rotating axle subjected to a steady and stationary load,

    - A non-rotating axle subjected to a non-steady but stationary load

    - A non-rotating axle subjected to a steady but non stationary load

    - None

- i

- ii

- iii

- iv

- v

Que. 9:	Threaded fasteners can be considered as a type of:

- Positive connection

- Force or friction connection,

- Material connection

- Negative connection

- None of the above

Que. 10:	The number 6.8 is found on the head of a standard bolt. The number 6 is related to:

- Yield strength

- Tensile strength

- Elastic limit

- Proof strength

- Non

Que. 11:	A double-threaded bolt has a pitch of thread of 20mm. What will be the axial advancement if the bolt is given a complete rotation without any axial load?

- 10mm

- 20mm

- 30mm

- 40mm

- None

Que. 12:	During the tightening process in a bolt connection, the driving torque is transformed into frictional. Which part of the frictional torque is involved in inducing the pre-stress force?

- Frictional torque between the mating of the internal and external threads.

- Frictional torque between head of bolt and clamped member.

- Frictional torque between nut and clamped member.

- Frictional torque between the two clamped members.

- None

An elastic element is to be introduced into this bolt connection, study the figure below and answer Questions 13 and 14.

Que. 13:	At what position will result in the bolt carrying a bigger load?

  - Position A only

  - Position B only

  - Position C only

  - At positions A and C

  - None

Que. 14:	At what position will result in the bolt carrying the lowest load?

- Position A only

- Position B only

- Position C only

- At positions A and C

- None

Que. 15:	Deep-groove ball bearings can take:

    - Axial loads only.

    - Radial loads only.

    - Longitudinal loads only

    - Axial and radials loads.

    - Axial, radial and longitudinal loads.

Que. 16:	Pre-loading of a ball bearing can increase the fatigue life of a bearing if done correctly. This advantage is due to its effect on:

    - Width of bearing

    - Height of bearing

    - Diametral clearance of bearing

    - Race conformity

    - None

Que. 17:	Pre-loading can be achieved by:

- Mounting bearing on a tapered shaft,

- Interference fit on the outer ring,

- Axially loading a bearing with a stick-out

- Fixing the bearing

- i., ii., and iii only

- ii., iii and iv only

- i., iii and iv only

- i., ii. And iv only

- None

Two angular-contact ball bearings (A and B) mounted as a duplex pair (back-back) has been axially pre-stressed on the inner rings. During operation, it is further subjected to an external operating load Pa. A force-displacement diagram of the situation is shown below, and the final condition of bearing A is indicated on the diagram.

Que. 18:	Determine the stickout from the Force-displacement diagram

- 40 µm

- 30 µm

- 20 µm

- 10 µm

- 0 µm

Que. 19:	What is the resultant force acting on bearing B?

- 4 kN

- 7 kN

- 10 kN

- 12 kN

- None (11)

Que. 20:	What is the magnitude of Fa?

- 3 kN

- 4 kN

- 6 kN

- 7 kN

- 8 kN

SECTION B

ALTERNATIVE A and B

The figure below shows a sectional view of a spur-gear transmission system consisting of a gear-box and a motor that have been built together. Within the gear-box there are four spur gears (1, 2, 3 and 4) mounted on three shafts. Gear 1 is mounted on the shaft of the motor and gear 3 is mounted on the output shaft.

Bearings A and B, and gears 2 and 4 are mounted on the transitional shaft.

The right figure shows the side view of gears 2 and 4 indicating the direction of the forces acting on them. The data below is provided.

Data:

Power:	4kW

Rotational speed of transitional shaft

Pitch diameter of gear 1:	40mm

Pitch diameter of gear 2:	160mm

Pitch diameter of gear 3:	50mm

Pitch diameter of gear 4:	150mm

Que. 1:	Assuming there are no losses involved in the transmission, determine the forces

Que. 2:	Determine the supporting forces acting on the transitional shaft on the respective planes. Show this in a drawing form.

Que. 3:	Provide a graph of the shear forces acting on the shaft in the respective planes.

Que. 4:	Provide a graph of the longitudinal forces acting on the shaft in the respective planes.

Que. 5:	Provide a graph of the bending moment load on the shaft in the respective planes

Que. 6:	Provide a graph of the torsional load on the shaft in the respective plane(s).

Take note of the continuation of Alternatives A and B

Continuation of Alternative A

Detail view of Z at Bearing B as well as the necessary data are provided below:

Fatigue concentration factor:	Kt

Material: St 50

| ultimate | Yield | Hardness | Fatigue limit or fatigue strength |  |  |  |  |  | Modulus |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | bS | HB | zSch | zW | bSch | bW | tSch | tW | E- | G- |
|  | zS |  |  |  |  |  |  |  |  |  |
|  | tS |  |  |  |  |  |  |  |  |  |
| 500 to 600 | 450 | 1400 to 1700 | 320 | 220 | 400 | 250 | 180 | 150 | 210000 | 80000 |
|  | 320 |  |  |  |  |  |  |  |  |  |
|  | 180 |  |  |  |  |  |  |  |  |  |

The table below gives the size factor b1 coefficient for combined bending and torsion:

| d mm) | 10 | 15 | 20 | 30 | 40 | 60 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b1 | 1.0 | 0.98 | 0.95 | 0.90 | 0.85 | 0.80 | 0.75 |

The table below gives b2 coefficient for surface finish:

| B | 300 | 400 | 500 | 600 | 700 | 800 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Polished | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Fine turning | 0.97 | 0.96 | 0.95 | 0.94 | 0.935 | 0.932 | 0.93 |

For the cut-section at x-x shown in the detail drawing above:

Que. 7:	Determine the nominal stress values at that section.

Que. 8:	Determine the effective stress values at that section.

Que. 9:	Determine the fatigue strength values.

Que. 10:	Determine b1 and b2.

Que. 11:	Provide the modified Goodman graph for that section.

Que. 12:	Indicate the stress state in the above modified Goodman graph.

Que. 13:	If failure can be as a result of both the mean and alternating stresses, indicate the load-line in the above graph.

Que. 14:	Determine the safety factor.

Continuation of Alternative B	20 marks

Assuming an additional load (axial load) of 1200 N is now introduced into the transitional shaft acting in a direction from A to B. The bearings A and B are both single row deep-groove ball bearings. And bearing B is designed to take axial loads.

Que. 7:	Give a sketch of the mounting of bearing A and B on the shaft.

Que. 8:	Is the state of loading on the bearings dynamic or static? Explain.

Que. 9:	For the bearing B, what value of the dynamic coefficient (fL) would you select for this type of transmission.

Que. 10:	Determine the equivalent load, P.

Que. 11:	Given that  as above, determine the rotational speed factor fn.

Que. 12:	Determine bearing capacity, C.

Que. 13:	Based on the capacity select a bearing for bearing B.

Que. 14:	Determine the nominal bearing life.

Que. 15:	Determine the modified bearing life, if the following data is available:

Operating temperature =100oC

Lubricating oil with viscosity of 46

Alternative C	20 marks

The figure below shows bolting of cylinder of an internal combustion engine. The cylinder head is held tight to the cylinder by four bolts.

Remark:	For the calculating of the resultant stiffness of the cylinder should be distributed uniformly on the four bolts.

Data:	Maxi combustion pressure: 100bar,	Distance across flats of bolt head =13mm

Mini combustion pressure: 0 bar	Internal diameter of cylinder = 80mm

E-modulus of Al-alloy: 74000N/mm2	Thickness of cylinder = 13mm

E-modulus of steel:	(24,000N/mm2) 210,000N/mm2

Coefficient of thermal expansion:	∝steel=11x10-6℃

ε=∆ll=∝.∆ϑ	∝Al=23x10-6℃

Designation on bolt head:	10.9

Table:	Fatigue strength σA for σm=σA	N/mm2.

| Metric grade | M4 - M6 | M8 - M16 | M18 - M30 |
| --- | --- | --- | --- |
| 12.9 | 150 | 140 | 130 |
| 10.9 |  |  |  |
| 8.8 | 130 | 120 | 110 |
| 6.9 |  |  |  |

Que. 1:	What is the maximum external force acting on a single bolt?

Que. 2:	What is the minimum external force acting on a single bolt?

Que. 3:	Provide an ideal curve of the external force-time curve.

Que. 4:	Derive an expression for the pre-stressed force if the resulting clamping force should be 1.5 times the external force.

Que. 5:	Provide a sketch of the bolted unit and indicate the effective point of action of the forces as well as the stress-flow.

Que. 6:	Determine the effective stiffness of the bolt and the clamped members.

Que. 7:	Determine the pre-stressed force.

Que. 8;	Provide a scaled force-displacement diagram of the system on a graph.

Que. 9:	Give an expression for determining the tightening torque required to yield the pre-stress force

Que. 10:	Determine the static and dynamic stresses within the bolt.

Que. 11:	Determine the static and dynamic equivalent stresses based on Distortion Energy Theory von Mises

Que. 12:	Determine the safety factor.

J. Aveyire

SOLUTION

Alternative A and B:

Que. 1

Que. 2

Sign conversion:

Alternative A continuation:

For section X-X:

Que. 7:	Nominal stresses:

Transverse shear:

Longitudinal tensile:

Bending tensile:

Torsional shear:

Equivalent stresses:	DET

Que. 8:	Effective stresses:

Que. 9:	Fatigue strength values:

Que. 10:

Alternative B Continuation:

Que. 7:

Que. 8:	Loading is dynamic, because the transverse force at x-x is static but the inner ring is rotating.

Que. 9:	fL=2

Que. 10:	Single row deep –groove ball bearing:

Que. 11:

Que. 12:	C=11.2kN	Bearing: 16006

Que. 13;	Bearing: 16006

Que. 14:

Que. 15:	From graph:

For L10 a1=1

Modified life Lna=a1a23L10=8,333.5 h

Alternative C

,

Que. 5

Serial connection:

Parallel connection:

Que.9:

Que 10:	Mean force Fsmean=Fpre +

STATIC:

DYNAMIC:

Que. 11	DET

Equivalent STATIC:

Equivalent DYNAMIC: