# KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY, KUMASI

COLLEGE OF ENGINEERING

FACULTY MECHANICAL AND AGRICULTURAL ENGINEERING

# B.Sc. (Engineering) First Semester Examination, 2009

Third Year

# AE 373 	MACHINE DESIGN 1

November, 2009.		 THREE HOURS

CONSTANTS:

SECTION A WILL BE COLLECTED AFTER 30 MINUTES

SECTION A	15 marks

Que. 1	A technical system whose primary objective is cause the flow and transformation of material can be classified as:

- Appliance

- Device,

- Equipment

- Machine,

- None

Que. 2	Agricultural “machines” are basically involved in the flow of:

- Energy,

- Material,

- Signal,

- Information,

- None

Que. 3	The output of the “Analysis of the Problem” is the Problem Statement. This consists of one or more of the following basic element(s)

- A statement of the design problem proper,

- Limitations placed upon the solution,

- The criteria of excellence to be worked to

  - Only (i) and (ii),

  - Only (ii) and (iii)

  - Only (i) and (iii)

  - All

  - Only (i)

Que. 4	The aim (purpose) of the design describes the intended use of the product solution-neutrally. Below are some technical products and their associated purpose. Which of the following purpose is not solution-neutral?

- Automobile:	Transport persons and goods,

- Photo-copier:	Replicate data,

- Lawn mower:	Mower the lawn,

- Lock on a door:	Protect against unwanted opening,

- Brake on bike:	Reduce or stop movement of bike.

Que. 5	Criteria are limitations or requirements placed on the technical product. Which of the following combination can be classified under criteria?

- Purpose of product,

- Maximum operating force,

- Size dimensions of product,

- Conceptual design of product

- i, and ii only

- i and iii only

- ii, and iii only

- ii, and iv only

- iii, and iv only

Que. 6	The profile of threads depends on its application. Basically, fastening screws have:

  - Trapezoidal,

  - Saw-shaped,

  - Round,

  - V-shaped

  - Square

Que. 7	The proof strength defines bolt grades. A metric grade of 6.8 is found on the head of a bolt. What will be the expected tensile strength?

- 6 N/mm2

- 60 N/mm2

- 600 N/mm2

- 6000 N/mm2

- None

Que. 8	Threaded fasteners can be considered as a type of:

- Positive connection

- Force or friction connection,

- Material connection

- Negative connection

- None of the above

Que. 9	A double-threaded V-shaped Metric bolt has a pitch of 1.5mm and pitch diameter (d2) of 9mm. What is the axial advancement of this bolt for one complete rotation (lead)?

- 1.5mm

- 3.0mm

- 4.5mm

- 9.0mm

- 13.5mm

Que. 10	The head of a cylinder has designed as shown below. The gas pressure acting on the head of the cylinder is as indicated. The resulting operational pressure force will appear to be introduced to the bolt head:

- Underneath the head of the bolt (Q) acting upwards,

- Underneath the head of the bolt (Q) acting downwards,

- At M acting upwards,

- At M acting downwards

- None

Que. 11:	During the tightening process in a bolt connection, the driving torque is transformed into frictional. Which part of the frictional torque is involved in inducing the pre-stress force?

- Frictional torque between the mating of the internal and external threads.

- Frictional torque between head of bolt and clamped member.

- Frictional torque between nut and clamped member.

- Frictional torque between the two clamped members.

- None

An elastic element is to be introduced into this bolt connection, study the figure below and answer Questions 12 and 13.

Que. 12	At what position will result in the bolt carrying a bigger load?

  - Position A only

  - Position B only

  - Position C only

  - At positions A and C

  - None

Que. 13	At what position will result in the bolt carrying the lowest load?

- Position A only

- Position B only

- Position C only

- At positions A and C

- None

Que. 14	Beneath is a concept design of an axle assemble. The load on the belt is steady and the axle is prevented from rotating. What type of loading will be acting on this axle?

    - Dynamic load,

    - Static load,

    - Dynamic and loads,

    - Fluctuating load,

    - None.

Que. 15	Failure of a component can be due to:

    - Fatigue by elastic cyclic loading,

    - Plastic deformation,

    - Excessive wear,

    - Pitting,

    - All above

SECTION B

Que. B1

Two shafts of a piston made of steel are connected together by means of bolting (Metric V-thread). The operating forces FA act directly beneath the head of the bolt and the nut as shown in the figure below.

Data:

Bolt:	8 pieces of M16x2

Distance across flats:	AF=24mm

Friction between thread:	µ=0.125

Friction at head of bolt:	µ=0.15

Metric grade:	10.9

Nut thickness:	m=16mm

Thickness of bolt head:	k=12.65mm

Que. 1:	Determine the stiffness (δb) of the bolt

Que. 2:	For the clamped parts, . Determine Ac using:

Que. 3:	Determine the stiffness (δb) of the clamped parts.

Que. 4:	Determine

Que. 5:	Determine the maximum operating force per bolt

Que. 6:	Determine the minimum operating force per bolt

Que. 7:	Given that the pre-stress force is 60kN, produce the force displacement graph.

Que. 8:	What is the maximum force present.

Que. 9:	What is the alternating stress acting in the bolt.

Que. 10:	Determine the mean stress acting in the bolt.

Qu.e B2

The figure below shows the views (Front and Left) of a transitional shaft mounted on two bearings A and B. Two spur-gears (2 and 4) mounted on this shaft are subjected to forces F2 and F4 respectively

The left view shows gears 2 and 4 as well the direction of the forces acting on them. The data below is provided.

Data:

Power:	4kW

Rotational speed of transitional shaft

Effective diameter of gear 2:	160mm

Effective diameter of gear 4:	150mm

Material for the shaft:	St 50

| ultimate | Yield | Hardness | Fatigue limit or fatigue strength |  |  |  |  |  | Modulus |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | bS | HB | zSch | zW | bSch | bW | tSch | tW | E- | G- |
|  | zS |  |  |  |  |  |  |  |  |  |
|  | tS |  |  |  |  |  |  |  |  |  |
| 500 to 600 | 450 | 1400 to 1700 | 320 | 220 | 400 | 250 | 180 | 150 | 210000 | 80000 |
|  | 320 |  |  |  |  |  |  |  |  |  |
|  | 180 |  |  |  |  |  |  |  |  |  |

The table below gives the size factor b1 coefficient for combined bending and torsion:

| d mm) | 10 | 15 | 20 | 30 | 40 | 60 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b1 | 1.0 | 0.98 | 0.95 | 0.90 | 0.85 | 0.80 | 0.75 |

The table below gives b2 coefficient for surface finish:

| B | 300 | 400 | 500 | 600 | 700 | 800 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Polished | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Fine turning | 0.97 | 0.96 | 0.95 | 0.94 | 0.935 | 0.932 | 0.93 |

Que. 1:	Assuming there are no losses involved in the transmission, determine the torque transmitted through the transitional shaft.

Que. 2:	Determine the forces  acting on gears 2 and 4.

Que. 3:	Determine the reacting forces acting on the transitional shaft at bearing A and B (on the respective planes). Show this in a drawing form.

Que. 4:	Provide a graph of the shear forces acting on the shaft in the respective planes.

Que. 5:	Provide a graph of the longitudinal forces acting on the shaft in the respective planes.

Que. 6:	Provide a graph of the bending moment load on the shaft in the respective planes

Que. 7:	Provide a graph of the torsional load on the shaft in the respective planes.

A detail view of Z is also shown above. The profile of the bearing and the cut-section at x-x are captured in this detail section:

Que. 8:	Determine the nominal stress values at that section.

Transverse shear:

Longitudinal tensile:

Bending tensile:

Torsional shear

Que. 9:	Applying DET, determine the equivalent stresses (static and dynamic)

Que. 10:	Determine the effective stress values at that section.

Que. 11:	Determine the fatigue strength values.

Que. 12:	Determine b1 and b2.

Que. 13:	Provide the modified Goodman graph for that section.

Que. 14:	Indicate the stress state in the above modified Goodman graph.

Que. 15:	If failure can be as a result of both the mean ad alternating stresses, indicate the load-line in the above graph.

Que. 16:	Determine the safety factor.

J. Aveyire

SOLUTION

Alternative A and B:

Que. 1

Que. 2

Sign conversion:

Alternative A continuation:

For section X-X:

Que. 7:	Nominal stresses:

Transverse shear:

Longitudinal tensile:

Bending tensile:

Torsional shear:

Equivalent stresses:	DET

Que. 8:	Effective stresses:

Que. 9:	Fatigue strength values:

Que. 10:

Alternative B Continuation:

Que. 7:

Que. 8:	Loading is dynamic, because the transverse force at x-x is static but the inner ring is rotating.

Que. 9:	fL=2

Que. 10:	Single row deep –groove ball bearing:

Que. 11:

Que. 12:	C=11.2kN	Bearing: 16006

Que. 13;	Bearing: 16006

Que. 14:

Que. 15:	From graph:

For L10 a1=1

Modified life Lna=a1a23L10=8,333.5 h

Alternative C

,

Que. 5

Serial connection:

Parallel connection:

Que.9:

Que 10:	Mean force Fsmean=Fpre +

STATIC:

DYNAMIC:

Que. 11	DET

Equivalent STATIC:

Equivalent DYNAMIC: