# KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY, KUMASI

COLLEGE OF ENGINEERING

FACULTY MECHANICAL AND AGRICULTURAL ENGINEERING

# B.Sc. (Engineering) First Semester Examination, 2010

Third Year

# AE 373 	MACHINE DESIGN 1

November, 2010.		 THREE HOURS

Attempt: All questions in SECTION A, and SECTION B

CONSTANTS:

SECTION A	30 marks

Que. 1	Appliance (device) is a technical system whose primary objective is primarily to cause the flow and transformation of:

- mass.

- energy.

- material.

- signal.

- forces

Que. 2	Which of the following combination of technical systems can be classified as appliance?

- Sieve and Turbine,

- Heat pump and Electric shaver,

- Sieve and electric shaver

- Turbine and heat pump

- Television and typing machine,

Que. 3	The output of the “Analysis of the Problem” consists of one or more of the following basic element(s)

- A statement of the design problem proper,

- Limitations placed upon the solution,

- The criteria of excellence to be worked to

  - Only (i) and (ii),

  - Only (ii) and (iii)

  - Only (i) and (iii)

  - All

  - Only (i)

Que. 4	As an engineer of a company, you have been approached by a client requesting for a system to enable him mill corn at a capacity of 500kg/hr that makes use of electrical power. A general formulation of the “Description of the aim” will be “To design a ”

- Machine to mill corn,

- Technical system to mill corn,

- Technical system to mill corn at a capacity of 500kg/hr,

- Technical system to mill corn at a capacity of 500kg/hr and it must make use of electrical power.

- None of the above is suitable.

Que. 5	The basic structure(s) (based upon the arrangements of the various elementary functional structures) is (are):

- Chain structure,

- Partial structure,

- Parallel structure,

- Cyclic structure

- i, and ii only

- iii and iv only

- i, ii, and iii only

- ii, iii, and iv only

- iii, iv and i only

Que. 6	Which of the load(s) listed below cannot be transmitted by an axle?

- Longitudinal loads,

- Transverse loads,

- Torsional loads,

- Bending moment

  - i,

  - ii,

  - iii,

  - iv,

  - i and ii only

Que. 7	When can an axle be said to be statically loaded?

    - A rotating axle subjected to a steady and stationary load,

    - A non-rotating axle subjected to a steady and stationary load,

    - A non-rotating axle subjected to a non-steady but stationary load

    - A non-rotating axle subjected to a steady but non stationary load

    - None

- i

- ii

- iii

- iv

- v

Que. 8	Threaded fasteners can be considered as a type of:

- Positive connection

- Force or friction connection,

- Material connection

- Negative connection

- None of the above

Que. 9	The number 6.8 is found on the head of a standard bolt. The number 6 is related to:

- Yield strength

- Tensile strength

- Elastic limit

- Proof strength

- None

Que. 10	A double-threaded bolt has a pitch of thread of 20mm. What will be the axial advancement if the bolt is given a complete rotation without any axial load?

- 10mm

- 20mm

- 30mm

- 40mm

- None

Que. 11	During the tightening process in a bolt connection, the driving torque is transformed into frictional. Which part of the frictional torque is involved in inducing the pre-stress force?

- Frictional torque between the mating of the internal and external threads.

- Frictional torque between head of bolt and clamped member.

- Frictional torque between nut and clamped member.

- Frictional torque between the two clamped members.

- None

Que. 12	Beneath is a concept design of an axle assemble. The load on the belt is steady and the axle is prevented from rotating. What type of loading will be acting on this axle?

    - Dynamic load,

    - Static load,

    - Dynamic and loads,

    - Fluctuating load,

    - None.

Que. 13	Failure of a component can be due to:

    - Fatigue by elastic cyclic loading,

    - Plastic deformation,

    - Excessive wear,

    - Pitting,

    - All above

Que. 14	The head of a cylinder has designed as shown below. The gas pressure acting on the head of the cylinder is as indicated. The resulting operational pressure force will appear to be introduced to the bolt head:

- Underneath the head of the bolt (Q) acting upwards,

- Underneath the head of the bolt (Q) acting downwards,

- At M acting upwards,

- At M acting downwards

- None

Que. 15	Which of the following design below of a gear-shaft connection is best?

- i

- ii

- iii

- All

- None

SECTION B

Que. B1:	15 Marks

Design the rectangular key for a shaft of 5O mm diameter. The compressional and shearing stresses for the key material are 42 MPa and 70 MPa.

Given: d=50 mm, , .

From Table, for a shaft of 50 mm diameter, b= 16 mm, h= 10mm, determine the optimal length

Que. B2:	55 Marks

The fatigue strength of a shaft is to be determined experimentally. At the test-setup, this shaft is supported on bearings. At the inlet part of this shaft power P is introduced at a rotational speed of n. There is no shear force at the inlet point. At the output end, friction brakes are applied. Additionally, the shaft experiences a centrifugal force due to unbalance mass m and a stationary force F.

As the shaft rotates, the resulting centrifugal force due to the unbalance mass will experience a change in direction. First in positive y-direction, then in the negative z-direction, then in the negative y-direction, then in the positive z-direction and back to positive y-direction for a complete rotation of the shaft.

Data:

P=20 kW

n=2000rev/min

F=2500N

m=0.5kg

N=107cycles

Geometry:

Material:

For b=20mm b1=0.95;	Surface finish=polished, b2=0.94 and

- How long must this shaft run on the test setup assuming a normal fatigue strength test is to undergo 107 cycles (rotations)?

- Provide graphs of the shear force and bending moment of the shaft in the x-y plane for the following position of the unbalanced mass:

- Mass m in positive y-direction,

- Mass m in negative z-direction

- Mass m in negative y-direction, and

- Mass m in positive z-direction

- The critical section of this shaft is identified to be at Z, where a groove has been cut. The detail of the groove is shown on the right side of the figure. What is the magnitude of the shear force, bending moment, longitudinal force and torque at Z as the shaft makes a single rotation. From positive y-direction (0o), through negative z-direction (90o), negative y-direction (180o), positive z-direction (270o) and back to positive y-direction (360o). Fill the table beneath

|  | Z-section |  |  |  |
| --- | --- | --- | --- | --- |
|  | Shear Force | Bending Moment | Longitudinal Force | Torque |
| m in positive y-direction (0o) |  |  |  |  |
| m in negative z-direction (90o) |  |  |  |  |
| m in negative y-direction (180o) |  |  |  |  |
| m in positive z-direction (270o) |  |  |  |  |
| m in positive y-direction (360o) |  |  |  |  |

- Plot graphs of shear force, bending moment, longitudinal force and torque at z-section against angle of rotation for one complete rotation:

- Determine the dynamic and static stresses caused by the above loads at Z.

- Determine the equivalent stresses based on Distortion Energy Theory.

- Provide the modified Goodman diagram (Smith Diagram) and indicate the state of z in the diagram.

J. Aveyire

AE 373 2010 SECTION B SOLUTION

Que.1	In order to determine the fatigue strength, the load cycle or rotation of N=107 is necessary. Therefore, for a rotation speed of n=2000rev/min:

Que.2	Graphs of:

Forces in x-y plane:

Case 1:	The centrifugal force, Fc, due to the unbalance mass acts in the positive y-direction

Case 2:	The centrifugal force, Fc, due to the unbalance mass acts in the negative z-direction

Case 3:	The centrifugal force, Fc, due to the unbalance mass acts in the negative y-direction

Case 4:	The centrifugal force, Fc, due to the unbalance mass acts in the positive z-direction

Sign conversion:

Forces in x-z plane:

Case 1:	The centrifugal force, Fc, due to the unbalance mass acts in the positive y-direction

No component.

Case 2:	The centrifugal force, Fc, due to the unbalance mass acts in the negative z-direction

Case 3:	The centrifugal force, Fc, due to the unbalance mass acts in the negative y-direction

No component.

Case 4:	The centrifugal force, Fc, due to the unbalance mass acts in the positive z-direction

:

Que.3	Table of Loads at Critical Section

Que.4	Graphs of SF, BM, LF, and Torque at Critical Section:

Que.5	Dynamic and Static stress caused by above loads

- SF:

- BM at Section:

- Torque:

- Stresses:

Bending stresses:

Compression/Tensile stresses:

Torsional stresses:

Shear stresses:

Que.6	Equivalent stress based on DET

Que.7	Modified Goodman diagram