KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY

COLLEGE OF ENGINEERING

DEPARTMENT OF AGRICULTURAL AND BIOSYSTEMS ENGINEERING

BSc. Agricultural Eng. and BSc. Landscape and Management

Second Semester 2019-2020

AE 452 Soil and Water Engineering II /AE 478 Soil and Water Conservation

May 2020

ASSIGNMENT C2

Instructions

Answer All Two questions and submit online on or before June 10, 2020

- Describe in detail any method that can be used to measure soil erosion on the field. (Times New Roman; Font 12; one and half spacing; Not more than 2 pages)

- The EI30 for a 120,000 m2 maize field was estimated as 23500. Given that the soil has 55 % silt + fine sand, 20 % sand (i.e. 0.10-2.0 mm), 1 % organic matter content and the soil structure is fine granular with slow permeability. If the slope of the land is 6 % with a length 120 m. Provided is erodibility nomograph, conservation practice and crop factor in Figure 1, Table 2 and 3 respectively.

Using the information provided above, Table 2 and 3 and Figure 1 determine using the USLE the following:

- Average soil loss if there is no conservation practice.

- Average soil loss if contour + strip cropping is practice.

- Given that the soil-loss tolerance value as 10 tons per acre, will the practice in (ii) be able to control erosion. Explain your answer.

- If the measures of contour + strip cropping cannot, then what conservation practice will be able to help control erosion

- One intends to break the slope into shorter length and use contouring to manage the land. What length would be most appropriate?

- Between the options of (iv) and (v) what is the best choice and why?

Table 2: Percentage slope and their corresponding conservation practice values

| SN | Percentage Slope | Conservation Practice Values |  |  |
| --- | --- | --- | --- | --- |
|  |  | Contouring | Contour strip cropping | Terracing and contouring |
| 1 | 1.1 – 2.0 | 0.6 | 0.30 |  |
| 2 | 2.1 – 7.0 | 0.6 | 0.32 | 0.10 |
| 3 | 7.1 – 12.0 | 0.7 | 0.35 | 0.12 |
| 4 | 12.1 – 18.0 | 0.8 | 0.40 | 0.16 |
| 5 | 18.1 – 24.0 | 0.9 | 0.45 |  |

Figure 1: Nomograph for erodibility determination

Table 3. Area covered by different crop types, and C-factor per crop type

| Sn | Crop type | C-factor |
| --- | --- | --- |
| 1 | Common wheat and spelt | 0.20 |
| 2 | Durum wheat | 0.20 |
| 3 | Rye | 0.20 |
| 4 | Barley | 0.21 |
| 5 | Grain maize – corn | 0.38 |
| 6 | Rice | 0.15 |
| 7 | Dried pulses (legumes) and protein crop | 0.32 |
| 8 | Potatoes | 0.34 |
| 9 | Sugar beet | 0.34 |
| 10 | Oilseeds | 0.28 |
| 11 | Rape and turnip rape | 0.30 |
| 12 | Sunflower seed | 0.32 |
| 13 | Linseed | 0.25 |
| 14 | Soya | 0.28 |
| 15 | Cotton seed | 0.50 |