AE 452 SOIL AND WATER ENGINEERING II

NAME: GYAPONG SEBASTIAN JNR

INDEX NUMBER: 2305314

ASSIGNMENT

ABOUT LIQUID – LEVEL SURVEYING INSTRUMENT

Liquid – level surveying instrument is used in laying out contours. This instrument is sensitive and can be operated by only two people.

This instrument comprises of the following below:

- length of transparent plastic tubing usually 6-10mm inside diameter and about 14m long

- Two poles of about 2metres height

- Four  rubber straps to attach the plastic tubing to the poles

- Muddy water of about 1-2litres is used as it is more visible in the tube

The plastic tubing is strongly attached to the poles using the rubber straps. The end of the tube should be 10cm from the top of each pole and the bottom fixing point about 20cm from the base of each pole. The plastic tube is then filled with muddy water until the level is about midway up each pole. Whenever the two poles are set, the force water surfaces in each plastic tube will be at same level.

NB: It is very important that, no air bubbles are trapped in the tube.