UNIVERSITY OF SCIENCE AND TECHNOLOGY,

COLLEGE 0F ENGINEERING

Final Year

AE 482 FARM MACHINERY DESIGN

Que. 1	The purpose describes the task of a technical product solution-neutrally:

Kind of product = f (purpose)

Provide the purpose of the technical systems in the table below.

| PRODUCT | PURPOSE |
| --- | --- |
| Car (not race car or special purpose car) |  |
| Lawn mower |  |
| Telephone |  |
| Power outlet |  |
| Drilling machine |  |
| Copier |  |
| Thermos bottle |  |
| Ball bearing |  |

Que. 2	For the construction of platform for a gari production plant, it was recommended that square-sectional cold rolled steel should be used as horizontal frames. There are two materials. St37-2 and St50-2, of that sectional type. Deduce the cost-strength relationship and determine the more suitable material on that basis. Given that:

| St 37-2 | 1.0 | 340 |
| --- | --- | --- |
| St 50-2 | 1.3 | 430 |

Que.3	A two cylinder two-stroke petrol engine has a cylinder bore of 100mm and a piston stroke of 130mm. The engine develops 53kW at 2500rpm when coupled to a dynamometer. At the same speed and with the fuel shut-off', the engine required a 9.3kW to motor it (to overcome friction) During the test when 53kW was developed the engine used 18.5lit/h of No.3 diesel fuel. The fuel contained 30MJ of heat per litre. Calculate:

i.	Engine indicated power.

ii.	Indicated mean effective pressure.

iii.	Net engine efficiency.

iv.	Indicated thermal efficiency, and

v. Rejected heat.

SOLUTION:

Que. 4

The sectional modulus of a solid square is:

|  | Kv | Stress | Kv/stress | Choice |
| --- | --- | --- | --- | --- |
| St 32-2 | 1.0 | 340 | 0.0029 | First |
| St 50-2 | 1..3 | 430 | 0.0030 | Second |

Que.5

Z=2, d=100mm, s=130mm, i(2-stoke)=1, Pb=53kW, n=2500rpm, Pf=9.3kW

  - Engine indicated power

  - Indicated mean effective pressure

  - Net engine efficiency

  - Indicated thermal efficiency.

  - Rejected heat