# KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY, KUMASI

INSTITUTE OF DISTANCE LEARNING

# B.Sc. (Engineering) Second Semester Examination, 2015

# AE 482 	FARM MACHINE DESIGN

MAY, 2015	TWO and HALF HOURS

ATTEMPT SECTIONS A and B

SECTION A

Atempt all Questions

Shade all Answers on Scannable Paper and Circle them on Question Paper.

1 mark for each question

Que. 1	A hydraulic pump can be classified as:

- Machine

- Device

- Equipment

- None

Que. 2	Which of the following combination of technical systems can be classified as appliance?

- Type-writer and Turbine,

- Heat pump and Mixer,

- Sieve and electric shaver

- Camera and heat pump

- Odometer and Car tail lamp

Que. 3	Which of the following listed below is not a type of requirement?

- Demand

- Explicit

- Wish

- Implicit

- None

Que. 4	The requirement:	‘Output of 7 lit/s	is a wish requirement.

- True

- False

Que. 5	The requirement:	‘Colour is blue’	is a wish requirement.

- True

- False

Que. 6	In determining requirements, the Associative Method was adopted and specifically the Main Heading method. Ergonomic data falls under which heading?

  - Pure technical

  - Law

  - Staff

  - Tool

  - Man environment

Que. 7	The purpose of a technical system is closely related to:

- Sub-function

- Subsidiary function

- Main function

- Tertiary function

- None

Que. 8	Which of the following basic structures can be found in Engineering Design Principles?

- Chain structure,

- Partial structure,

- Parallel structure,

- Cyclic structure

- i, and ii only

- iii and iv only

- i, ii, and iii only

- ii, iii, and iv only

- i, iii and iv i only

Que. 9	A function gives information about the activity

  - True.

  - False.

Que. 10	Under the service requirement of an engineering material, one talks about its:

    - Ease of joining the parts

    - Ease of shaping the part

    - Cost

    - Strength

    - None

Que. 11	Sand belongs to what group of engineering materials?

- Organic

- Ferrous

- Ceramic

- None

Que. 12	A Tough material is required under which manufacturing process?

- Machining

- Welding

- Casting

- Bending

- Rolling

Que. 13	Which of the statements below is true about corrosion?

- A physical reaction with other parts,

- A non-chemical reaction with the environment,

- Corrosion can be observed only on metals,

- Corrosion can be observed in both metals and non-metals,

- None of the above is true.

Que. 14	The relative material cost gives an impression about the:

- Quantity of the material,

- Quality of the material,

- Both quality and quantity of the material,

- Efficient utilisation of the material,

- None of the above.

Que. 15	The direct cost of a material may be influenced by:

  - Quality of material,

  - Quantity of material,

  - Efficient utilization of the material

Which of the following combination is correct?

    - i. and ii. Only.

    - i. and iii. Only.

    - ii. and iii. Only.

    - All of the above.

    - None of the above.

For Que. 16 to Que 18

For the construction of a platform for a TV, it was recommended that round-sectional cold rolled steel should be used as horizontal canti-lever frames. There are two types of materials available, St32-2 and St50-2. The member is subjected to pure bending. Given that the section modulus under bending is  and Table 1

| St 32-2 | 1.0 | 340 |
| --- | --- | --- |
| St 50-2 | 1.3 | 430 |

Que. 16	Determine the relationship for the volume of this part

- None

Que. 17	Determine the cost-strength relationship based on .

- None

Que. 18	From the above relationship, which material has the highest cost-strength value?

- St 32-2

- St 50-2

- Both A and B are the same

Que. 19	The life-span of a technical system may depend on whether the design was based on either the shot-time or long-time strength as shown in Figure 1.

Figure 1:	Load cycle

If the square-sectional frame in Que 16-Que 18 had been designed based on its long-time strength, which of the following statements is true in case it is re-designed based on its short-time strength to carry the same load?

- Its dimension will increase

- Its dimension will reduce

- Its dimension will remain unchanged

- Its surface must be polished

- None is true

Que. 20	The determination of the brake power of a four cylinder, four-stroke engine with a prony brake dynamometer yielded the following results: Brake power (50kW), rotational speed (2200rpm) and length of arm (0.955m). The engine had a bore of 120mm and a stroke of 100mm.

Calculate the brake constant.

- 1x10-4

- 5.2x10-3

- 2.2x10-1

- 2.3x10-1

- None

Que. 21	Compute the length of a new prony brake dynamometer when used on the engine in Que. 20. The reading of the force was 217N.

- 1.937m

- 1.001m

- 0.100m

- 0.011m

- None

Que. 22	Determine the engine brake mean effective pressure of the engine in question 20.

- 301,582.7 N/m2

- 603,165.4 N/m2,

- 723,798.5 N/m2,

- 2,412,661.6 N/m2

- None

Que. 23	Compute the engine brake torque of the engine in Que. 20.

- 681.82 Nm

- 434.28 Nm

- 217.14 Nm

- 21.71 Nm

- None

For the above combustion engine in Que 20, the indicated thermal and brake thermal efficiencies are 75% and 80% respectively.

Que. 24	Compute the fuel equivalent power

- 40 00kW

- 62.50 kW

- 66.67 kW

- 83.33 kW

- None

Que. 25	Given that the fuel equivalent power Pfe, is 85kW and gross heating value of diesel is 45585.68kJ/kg, determine the fuel consumption rate (kg/h).

- 11.19

- 6.71

- 1.86

- 0.67

- None

Que. 26	Compute the rejected heat, assuming the fuel equivalent power is 85 kW.

- 63.75 kW

- 34.00 kW

- 21.25 kW

- 17.00 kW

- None

Que 27 to Que 29

A milling machine is to be driven by a prime mover. The characteristic curves of two prime movers “a” and “b”and that of the load (milling machine) are provided in Figure 2.

Figure 2:	Characteristic curves

Que. 27	The most suitable prime mover for this load is:

- Characteric curve a,

- Characteristic curve b,

- Both curves a and b

- None of the curves

Que. 28	Pairing of the load and prime mover ‘a’:	An increase in the torque of the load above the operating torque is accompanied by:

- An increase in the torque of prime mover a

- A decrease in the torque of prime mover a

- Neither  increase nor decrease in the torque of prime mover a

Que. 29	Pairing of the load and prime mover ‘b’: An increase in the torque of the load above the operating torque is accompanied by

- A prime mover torque greater than corresponding load torque

- A prime mover torque less than corresponding load torque

- Neither  increase nor decrease in the torque of prime mover b

For Que. 30 to Que. 32, make use Figure 3

Figure 3:	Characteristic curve of an internal combustion engine

Que. 30	Along the torque curve from C to D, the torque increases as the speed decrease because the friction torque declines with decrease in speed

- True

- False

Que. 31	Along the torque curve from E to D, the torque increases as the speed increases because the amount of fuel supplied to the cylinder increases as the speed increases.

- True

- False

Que. 32	Along the torque curve from E to D, the torque increases as the speed decreases because the friction torque increases with increase of speed

- True

- False

Que. 33	SAE 10W-30 signifies:

- Half-grade

- Full-grade

- Single-grade

- Double-grade

- Multi-grade

Que. 34	Given that:

Derive the relationship existing between these quantities in the case of a hydraulic fluid

Que. 35	Hydraulic system diagrams can be provided in the following form

- 3-D diagrams

- Cutaway diagrams

- Schematic circuit diagram

- a and b only

- b and c only

- a and c only

- All above

- None of the above

Que. 36	Which of the following factors affect wear?

- Type of motion (sliding, rolling, impact etc.)

- Trend of motion (continuous, intermittent, reversible, oscillation)

- Forces/stresses (compression, tensile, etc)

- Speed

- Temperature

- Duration or distance travelled

- Properties of material

- Form and nature of contact surfaces eg concave-convex surfaces, roughness

- All

- All except b, and h

- Only c

- Only a, c, e and g

- None of the above

Que. 37	Wear is the progressive lost of material on the surface of a solid body. Cavitation is therefore a wear process. Under which type of mechanism of wear can caviatation be classified?

- Adhesive wear

- Abrasive wear

- Surface disruption (surface fatigue)

- Tribo-oxidation (oxidative wear)

- None of the above

For Que. 38 to Que. 50, Figure 4 must be considered.

Figure 3:	Hydraulic circuit

Que. 38	Identify the component labelled ‘a’.

- Pump

- Filter

- Tank

- Reference line

- Connecting hose

Que. 39	Identify the component labelled ‘c’.

- Reducing valve

- Relief valve

- Check valve

- Sequence valve

- Unloading valve

Que. 40.	Identify the component labelled ‘d’.

- Storage tank

- Linear pump

- Linear motor

- Double acting cylinder

Que. 41	Identify the component labelled ‘e’.

- Spool valve

- Pre-fill valve

- Pilot valve

- Shuttle valve

- Check valve

Que. 42	Identify the component labelled ‘f’.

- Shuttle valve

- Pilot valve

- Orifice

- Throttle

- None

Que. 43	Identify the component labelled ‘g’.

- 3/4 way open-centre valve

- 4/3 way open-centre valve

- 3/4 way tandem-centre valve

- 4/3 way tandem-centre valve

- None

Que. 44	Determine the path of flow of the hydraulic fluid if the spool is displaced to the left.

- Y-A-b-f-B-Z

- Y-B-f-b-A-Z

- Y-B-e&f-b-A-Z

- Y-A-h-f-B-Z

- None

Que. 45	One of the under listed components below is governed by the equation:

.

- Throttle

- Orifice

- Reducing valve

- Relief valve

- None

The setup is in operation whereby the fluid flows from Y-A-h-f-B-Z. The following data is provided:

Pressure of pump=8 bar

Atmospheric pressure=0 bar

Diameter of the shaft of the piston:

Internal diameter of cylinder:

Que. 46	Determine the pressure P1 .

- 0 bar

- 981 N/A1

- 4 bar

- 8 bar

- None

Que. 47	Under what condition will the piston within the cylinder travel out with a uniform velocity?

- None

Que. 48	Determine the pressure force acting on the surface A1 of the piston.

- 2492.5 N

- 3925.0 N

- 24925.3 N

- 99701.3 N

- None

Que. 49	Determine the pressure force F2 acting on the surface A2 of the piston, if p1=10.00 N/mm2 (uniform velocity).

- 991.0 N

- 3925.3 N

- 25269.4 N

- 30175.7 N

- None

Que. 50	Determine , for F1=1,000,000 N and F=600,000 N.

- 152.4 N/mm2

- 228.6 N/mm2

- 609.5 N/mm2

- 1,223.0 N/mm2

- None

SECTION B

Section B:	Attempt EITHER Question 1 or Question 2.

Provide answer in the ANSWER BOOKLET

Question 1:

You are required to design a shaft for a cassava grater. There were two materials available St37 and St50. The required cross-section of the shaft is circular and the shaft is expected to be subjected to only torsional load. Table 1 contains some valuable data required for  this task, make sure you select the right data.

Table 1:	Data for St37 and St50

| St 37 | 1.0 | 140 | 190 | 240 | 210000 |
| --- | --- | --- | --- | --- | --- |
| St 50 | 1.3 | 180 | 250 | 320 | 210000 |

Where;

static torsional strength

Fatigue limit for bending

Tensile yield strength

- State the reference material,

- Deduce the cost-strength relationship,

- Determine the suitable material on the basis of cost-strength,

- The cost of production of this shaft with the reference material was Gh¢250.00, what will the cost of production of this very if the other material was selected?

In the design of a platform, series of round bars were required to form a horizontal parallel grid onto which metal sheets were placed. The platform is subjected to unifom distributed load w. The design demands that the deflection not to exceed a certain value:

fmax=5.w.l4384. E I,          I=πd464  for solid rod bar

- Derive an expression between cost-maximum deflection,

- What property of the material can give rise to change in cost for a certain deflection.

Question 2:

From the view point of production of the bearing house there were two possible manufacturing processes (based upon the technical skill and the equipment available within the workshop) The total cost (C) verses the number of bearing houses (n) produced obeyed the following equations:

- Casting :	Cost = 500 + 6n

- Cut & weld	Cost = 200 + 10n

n is the number of product

  - At what value of n would there be no difference in the cost/bearing irrespective of the manufacturing process selected.

  - If n = 50, which the manufacturing processes will you choose? Prove it.

A six-cylinder, gasoline engine operates on the four-stroke cycle. The bore of each cylinder is 80 mm and the stroke is 100 mm. At the speed of 4100 rpm the torque developed at the flywheel is 160 Nm and the fuel consumption is 5.5 gm/sec. [or 19.8 kg/hr.) with a gross heating value of Hg=44000 kNm/kg.

At the same speed of 4100rpm and with the fuel shut-off', the engine required a 8.0kW to motor it (to overcome friction)

Calculate:

- Brake power,

- The brake mean effective pressure,

- Brake thermal efficiency if the calorific value of the fuel is 44000 kJ/kg and

- Indicated power.

- Rejected heat

J. Aveyire