KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY

COLLEGE OF ENGINEERING

DEPARTMENT OF AGRICULTURAL AND BIOSYSTEMS ENGINEERING

CONSTRUCTION OF A PIG FARM FOR 300 ADULT PIGS AND 700 PIGLETS

AE 453

GROUP ONE

GROUP MEMBERS

WIAFE EVANS KOFI                                            6033816

YARO PHILIP YARO                                             6029816

KORANTENG AYERAKWA ISAAC                    6029516

OPOKU AMA GIFTY                                             6031616

ASAMOAH SYLVESTER                                      6025916

NII ADOTEY ADDO                                              6023416

MENSAH ISAAC BENJAMIN KOFI                     6030216

OFOSU ERNEST NANA ARKO                            6031516

COBBINAH JUDAH PRINCE                                6027316

# 1.0 Introduction

## 1.1 Background study

Pig farming or swine production is a branch of animal husbandry where domestic pigs are raised and bred for the purpose of food. They are highly valued as a source of meat and fat, this type of livestock is mainly raised for human food but may also supply skin, fat and other materials for clothing, ingredients for processed foods, medical use and cosmetics.

Breeds of pigs commonly reared includes landrace, duroc, Yorkshire, etc. mostly referred to as the exotic breeds of pigs. Pig farming or production maybe done in three (3) ways; intensive commercial units, commercial free range enterprise or extensive farming. For the purpose of this project, we consider the intensive commercial unit or housing system due to the large number of pigs to be raised and its being very cost-efficient in this instance. Also this system feed is readily available and provides a greater degree of protection for the pigs.

## 1.2 Objective

The main objective of this project is to construct a housing unit for 1000 pigs (300 adults and 700 piglets)

Other objectives of this project includes;

- Providing a hazard free environment for the pigs (regulated ventilation, appropriate spacing and protection against rain and direct sun).

- Applying scientific management practice

- Providing safety from attacks.

# 2.0 Materials

The materials to be employed for the housing unit are concrete blocks for the base of the housing, steel for the columns, doors and door frames and aluminum sheets for roofing.

The structure for storage and mixing of feed would be present outside the housing unit of the pigs.

# 2.0 SPACING AND VENTILATION

## 2.1 Spacing

Total number of pens = 16 for both adults and piglets

Adult pens = 6

Piglets pens = 10

Number of adult pigs per pen =  3006 = 50 pigs.

Number of piglets per pen = 70010 = 70 piglets

ADULT PIGS

Area of pen = 4.50m × 8.0m

Nominal area of  an adult  pig = 4.50 × 8.050 = 0.72m2 per pig

PIGLETS

Area of the pen = 3.5m × 8.0m

Nominal area of a piglet =   3.50 × 8.075 = 0.373m2 per piglet.

## 2.2 Ventilation System

We are depending on natural ventilation through the wide open spaces(windows) created on all sides of the structure. This was done to reduce cost involved in putting up the housing unit.

Clay soil would also be used to linen the inner walls of the housing unit to help maintain a lower temperature, hence keeping the pens or rooms cool at all times.

# 3.0 EQUIPMENT

- Bulb per pen for lighting

- Feeders for hosting/containing feed

- Drinkers for hosting water

# 4.0 Pictorial drawings of the housing system

Pictorial view of the structure

Front elevation

floor plan of the structure

# 5.0 JUSTIFICATION

The structure is divided into twenty partitions, 6 equal partitions for adult pigs and 10 set of partitions for the piglets. For the other 4 chambers, 2 being used for farrowing and weaning and the other 2 serving as a storage for feed and other items. There would be 50 pigs per pen for the adults and 70 piglets per pen. Steel columns and beams would be used to support the whole structure. Clay soil included in the construction to provide a low room temperature. Drains are always present in all pens to facilitate easy cleaning of the pens.

# 6.0 CONCLUSION

Considering the structural plan, both the adult pigs and piglets will be free from predators they will also have free access to feed and enough space for free movement within their various pens. Necessary design plans have been implemented to cater for enough ventilation in the structure. Drains have been provided for easy disposal of waste. Also enough lightening has been provided for night activities.