NAME : BENJAMIN  DONKOR

INDEX NO. : 9813213

COURSE CODE : AE 451 (IRRIGATION AND DRAINAGE)

ASSIGNMENT

QUESTION 1

A GRAPH OF INFILTRATION RATE AGAINST TIME

(b)

QUESTION 2

From the first graph the Kostiakov and Philips equations for infiltration rate can be generated as

Kostiakov Equation

i=ktn

i = 2.9413t-0.33

From graph;

K = 2.9413;    n = -0.33   and

r2= 0.9623

Philips Equation

i = St-0.5 + ksat

i = 2.9526t-0.5 + 0.4

From graph;

r2= 0.9521

QUESTION 3

Using Kostiakov equation; at t = 180 min

i = 2.9413t-0.33

i = 2.9413(180)-0.33

i = 0.53 mm/min

QUESTION 4

D = kn+1tn+1

D = 2.9413(-0.33+1)t(-0.33+1)

D = 4.39t0.67

QUESTION 5

From Philips equation,

S = 2.9526 md-0.5

ksat = 0.4 md-1

QUESTION 6

From the cumulative infiltration equation;

D = 4.39t0.67

If D = 75mm; then

t = (D4.39)10.67

t = (754.39)10.67

t = 69.135 min

t = 1.15 h