A crankshaft can be considered as the heart of an I.C. engine, without which it cannot work. It allows the pistons to continuously reciprocate inside the cylinder using the unbalanced masses called crank webs. It has an intricate solid geometry.

The crankshaft consists of three main parts namely the crank pin, crank web and shaft. The big end of the connecting rod is connected to the crankpin; the crank web connects the crankpin to the shaft portion which is rotated by the main bearings and transmits power to the outside source through the belt drive, gear drive or chain drive. Load of the gas forces inside the combustion chamber is transferred and distributed over each crankpin through the connecting rod. Every crank web is subjected to bending moment and twisting moment. There is a flywheel attached at the end of the crankshaft to bring uniformity in torque of a four-stroke I.C. engine by storing energy during the power stroke and releasing the same during the other three strokes.

The crankshaft should have sufficiently high strength to sustain the gas force acting vertically downwards during the expansion stroke. The strength of the crankshaft should be high enough to avoid bending failure. Hence, the crankshaft greatly affects the life and reliability of an I.C. engine.

The following stresses directly affect the crankshaft of an I.C. engine:

1. Bending Stress: - The gas force generated by the burning of the air-fuel mixture in the combustion chamber, above the piston head, forces the piston downwards. This force is transmitted to the crankpin bearing. It is a bending load, causing corresponding bending stress.

2. Torsional Shear Stress: - Crankshaft is a rotating component, and it runs at high speeds. Any rotating mass generates centrifugal force greater the engine speed, the greater the centrifugal force. Connecting the rod also generates centrifugal force. Due to this, both, bending and torsional shear stress are developed in the crankshaft. These areas