NAME: AGBANYO DANIEL KOFI			DATE: MAY 14, 2022.

INDEX NUMBER: 3614218

PROGRAMME: BSC MARINE ENGINEERING

COURSE: SHIP MACHINERY SYSTEM DESIGN (MME 473).

MID-SEMESTER ASSIGNMENT

- Case hardening is a process by which a metal surface is strengthened by forming a thin layer of a harder metal alloy over the softer core of the original metal. Case hardening is a material processing method this is used to increase the hardness of the outer surface of a metal. Solid metals contain molecules and atoms packed tightly into crystalline structures. When carbon diffuses into the crystalline structure from carbon-bearing matter on the surface of the low carbon steel or alloy steel, the outer layer’s strength and hardness increase dramatically. The case hardening method aims at overcoming the problem of brittle mass formation.

Advantages of case hardening are discussed below:

- Improves the durability and usage economy of steel parts.

The mechanical strength and surface hardness developed by case hardening and the retention of the softcore significantly enhance the wear resistance and fatigue life of the components. Retention of a softer core improves the ability to absorb the energy released by impact loads and thus contributes to longevity and economic advantage.

- Enables the use of steel with superior machinability for heavy-duty applications.

Alloys used for heavy-duty applications have low machinability because they must be harder and stronger. Case hardening process enables the use of low carbon steel with precision machinability for weapons and firearms applications and other similar heavy-duty uses that require mechanical strength, fine finishes, and accurate geometry. Subsequent case hardening after machining provides superior wear resistance and hardness on the surface of the precision machined parts.

- Enhances the performance of taper roller bearings.

Case hardening leaves a softer core that is advantageous for accommodating a misalignment condition common in taper roller bearings. A strong surface layer with a soft inner core ensures fracture resistance under an extreme operating environment that includes misalignment, vibration, and severe shock loads

- Enables formation of residual compressive stresses in gear teeth

Case hardening on gears enables the formation of residual compressive stresses both inside the case depth and on the surface, which avoids crack formation and neutralizes bending stress. Advanced induction hardening methods have enabled accurate case hardening of heavy-duty spiral bevel gears, pinion, hypoid gears, and helical gears of complex geometry.

- Weldability of steel is improved by case hardening

Case hardening improves the weldability of steel, which is important for certain engineering applications.

- Minimizes Distortion

Case hardening produces a low-cost, wear-resistant load-bearing surface with minimal distortion.

- Produces improved fatigue resistance

There are different processes of case hardening.

- Carburizing

Carburizing is one of the types of the heat treatment process and it is also known as carburization. In the carburizing technique, the metal is heated in the presence of charcoal or carbon monoxide. Carburizing is to make the metal harder by heating it in a very short interval of time. In the process of carburizing, various hardened metals are used like carbon monoxide, barium carbonate, sodium cyanide, etc.

After completing carburizing, you may see the changes in size and shape of the original metal. But these changes are very small compared to other heat treatment processes.

- Nitriding

Nitriding is one of the heat treatment processes in which nitrogen spread over the surface of the metal to form its layer on the surface of the metal. Nitriding is one of the parts of the case hardening. It is used mostly in the case of steels that contain low carbon content. There are some expectations to the previous like titanium and molybdenum. Nitriding has a very larger number of applications like it is used in making gears, injectors, screws, camshafts, and various tools.

Nitriding is divided into three main processes :

  - Plasma nitriding

Plasma nitriding is mostly used in industrial applications. Plasma nitriding is also termed ion nitriding or glow-discharge nitriding. In plasma nitriding, an effective electric field is generated which results in the ionization of the gas molecules. In the entire process of plasma nitriding, nitrogen is used. It is the perfect process for treating steel. In the plasma nitriding process, along with the nitrogen certain amount of hydrogen and argon are also used.

  - Gas nitriding

In gas nitriding, ammonia is used. During gas nitriding, ammonia comes in contact with heated metal and this ammonia separates into hydrogen and nitrogen. The nitrogen formed then spread on the surface of the metal forming nitride layer.

  - Salt bath nitriding

In the salt bath nitriding, the cyanide salt plays an important role. The temperature is between the 550-570º C. The most beneficial part of the salt bath trading is that you get the diffusion of the cyanide salt over a larger surface area than any other process in the same time interval. This process is done very quickly and requires only 4 hours. Along with the quick process, there are other complications involved in this process.

- Carbonitriding

Carbonitriding is the type of the case hardening process in which the hardness of the metal surface is increased and also any type of deformation is also reduced during this process. As the name suggests, in carbonitriding, the carbon and nitrogen atoms spread over the metal surface and result in increasing the hardness of the metal. The carbonitriding is carried out above 800º C. The layer that forms on the workpiece as a result of the carbonitriding is quite hard. This layer is of average thickness ranging from 0.07 mm to 0.5 mm. The maximum thickness of the layer that can be formed is 0.75 mm.

- Cyaniding

Cyaniding is a case hardening process in which the metal is heated in the temperature range of 871 to 954º C. Here, the metal part is heated in the presence of sodium cyanide. Cyaniding results in the formation of a layer of thickness ranging from 0.25 mm to 0.75 mm. This process is typically applicable for low-carbon steels. Also, unlike other processes, it requires only 20 to 30 minutes. Its application is found on metallic parts such as gears, screws, small nuts, etc. The disadvantage of this process is that here cyanide salt is used and this cyanide salt is very poisonous and this hazardous to our life. On the other side, its advantage is that it is an efficient and fast process.

- Ferro Nitrocarburizing

Ferric nitrocarburizing is one of the case hardening processes in which the carbon and nitrogen are spread out on the ferrous metals. . Ferric nitrocarburizing is carried out at the sub-critical temperatures during the process of the salt bath. This process is practiced mostly in the temperature range of 525ºC to 625º C. During such temperatures, the steels, and other alloys are in the ferritic phase.

Examples of components made from case hardening are :

- Gears

- Fasteners

- Camshaft

- Rods

- Pins

A material's strength and stiffness are two physical qualities. One significant distinction is that stiffness refers to an object's capacity to bear stress without breaking. On the other hand, strength is an object's ability to resist deformation when under stress. A material's strength and stiffness are two physical qualities. One significant distinction is that stiffness refers to an object's capacity to bear stress without breaking. On the other hand, strength is an object's ability to resist deformation when under stress. If the applied force exceeds the material's tensile strength, the material will break. The sturdy material maintains its shape. If the applied force is greater than its strength, it simply breaks, losing its original shape. Stiff materials with little strength, on the other hand, will deflect. The deflection aids it in accommodating the force applied. As a result, if the force is withdrawn, it can revert to its former shape.

A ship structure is sufficiently strong if it can support all of the fixed loads of its machinery, fittings, and equipment, as well as all of the cargo loads, and if it can withstand all of the hydrostatic and hydrodynamic loads that can be imposed on it in calm water or in any kind of seaway in which it is supposed to operate without permanent change of shape, cracking, or fracture. It is sufficiently stiff if, under any of these service loads, it does not bend or deform excessively to interfere with equipment alignment or any other major or minor function.

When generating full thrust, a bronze screw propeller on a high-powered ship with excessively thin blades may bend so much that its shape and propulsive qualities are altered. The propeller is not strong enough if the blades are permanently bent during emergency maneuvering. If local components of the structure are in resonance with the periodically applied forces, vibratory motion can build up to intolerable peaks. As a result, stern rudder and shell plating must be reinforced to control panting or pulsating deflections caused by propeller forces. The bow shell is also vulnerable to panting from wave-impact loadings, necessitating the addition of extra stiffeners known as panting stringers.

- A crankshaft is the heart of an internal combustion engine, without which it cannot function. It uses unbalanced masses called crank webs to allow the pistons to constantly revolve inside the cylinder. Its solid geometry is complex.

The crankshaft is made up of three primary components: the crankpin, the crank web, and the shaft. The connecting rod's big end is attached to the crankpin, and the crank web connects the crankpin to the shaft part, which is rotated by the main bearings and sends power to an external source through a belt, gear, or chain drive. Load of the gas forces inside the combustion chamber is transferred and distributed over each crankpin through the connecting rod. Every crank web experience bending and twisting moments. A flywheel is mounted to the end of the crankshaft to provide torque uniformity in a four-stroke internal combustion engine by collecting energy during the power stroke and releasing it during the other three strokes.

The crankshaft should be strong enough to withstand the vertical downward force of the gas during the expansion stroke. The crankshaft's strength should be sufficient to prevent bending failure. As a result, the crankshaft has a significant impact on the longevity and durability of an I.C. engine.

The crankshaft of an I.C. engine is subjected to the following stresses:

    - Bending Stress: The gas force created by the combustion of the air-fuel mixture above the piston head forces the piston downward. The crankpin bearing is the recipient of this force. It's a bending load that results in bending stress.

    - Shear Stress in Torsion: - The crankshaft is a spinning component that spins at a high rate. Centrifugal force is generated by any rotating mass. The higher the engine speed, the larger the centrifugal force. Centrifugal force is also generated by connecting the rods. The crankshaft develops bending and torsional shear stress as a result of this.