KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY

KUMASI, GHANA

COLLEGE OF ENGINEERING

DEPARTMENT OF MECHANICAL ENGINEERING

BSc. MARINE ENGINEERING

MME 462

MARINE CONTROL ENGINEERING AND AUTOMATION

IDENTIFY ALL THE VALVE IN MARINE SYSTEMS, SKETCH FIVE OF THEM AND GIVE DETAIL APPLICATION OF THE VALVES.

MAY, 2023.

GROUP MEMBERS

- AMOAKO BERNARD KOFI                  	8310519

- BLANKSON ISAAC KWEKU          	 	8310819

- KORMASHIE  EDEM      		  	 	8311219

- AKUMANE JOSHUA      		   	 	8310319

- ADDO DANKWA GODFRED 			8309819

- ACHEAMPONG ANDY KOFI      		8309719

VALVES

Ships need valves to regulate the pressure and flow of fluids in the various onboard systems.  Valves can effectively manage the transfer of liquids, steam, gases, and slurries. They are also important for regulating pressure and flow in various piping and machinery systems on marine vessels. Each type of valve serves one of more of the following purposes,

- Stop/start flow

- Reduce/increase flow

- Control direction of flow

- Regulate flow or pressure

- Relieve pressure from pipe systems

COMPONENTS OF A VALVE

Valves are made up of several different components.

- First is the valve body (shell), which holds all of the components together.

- There is a bonnet, which covers the opening.

- Valve trim refers to the internal parts.

- Valve disks are designed to throttle or stop flow

- Seal rings provide a seat for the disks

- Valve stems help control the closing of the valve

TYPES OF VALVES USED ON MARINE VESSELS

The following are the major types of valves used on marine vessels:

- Gate valves

- Butterfly valves

- Globe valves

- Relief valves

- Pinch valves

- Piston valves

- Ball valves

- Diaphragm valves

GATE VALVE

Gate valves are one of the most common types of valves used on any type of ship. They control the flow of liquid through the pipes by raising or lowering, just like a gate. As the name suggests, a gate valve consists of a simple mechanism called as “the gate” or the valve disc, which performs the main function of regulating the flow of the fluid. However, it is to note that the gate valve can have only no-flow or full-flow condition and thus can only be operated in one position. The valve gives a full-bore flow without change of direction. There are two types of gate valves known as:

- Rising stem type

- Non-rising stem type

OPERATION OF A GATE VALVE

The working of the gate valves is pretty simple as it does not involve any complex mechanism. The spindle wheel, which is attached to the spindle rod, is rotated to move the “gate” at right angle to the flow of the fluid. The screwed spindle works in a nut and lifts the valve to open or close the “gate” between the circular openings furnished with seats. The valves and the seats may be either tapered or parallel on their facing sides.

USES OF A GATE VALVE

- Gate valves are used in applications requiring minimum pressure loss.

- They are also used in applications wherein bidirectional flow is required.

BUTTERFLY VALVE

Butterfly valves are used to isolate, start, stop and throttle the flow of fluids. Its construction may vary, but a butterfly-type disc and sealing are common in every valve. Moreover, it is easy to maintain due to fewer parts and offers low-pressure drop and high-pressure recovery. When the valve is open, the disc becomes parallel to the direction of flow and allows the fluid to flow. While in the closed state, it is perpendicular to the pipe wall and blocks the flow.

Small-sized butterfly valves are operated by a lever, while a hand wheel is used to actuate large butterfly valves. They are small in size but can handle a large flow of fluids at low pressure and low temperature. Unlike a ball valve, a butterfly valve can also manage slurries with a high ratio of suspended solids. As they are small in size, they require less space for installation as compared to gate and globe valves.

USES OF BUTTERFLY VALVE

- Butterfly valves are used where space is limited.

- Butterfly valves can be used for throttling or regulating flow as well as in the full open and fully closed position.

GLOBE VALVE

Glove valve is commonly used on board ships in places such as bilge suction lines. A linear motion valve, which regulates the flow of the fluid, the valve has a globe (bulbous) shaped body that houses the valve seat and the disc. It consists of a spherical body divided into two halves by an internal baffle. The baffle diverts the flow of fluid, which ultimately drops the pressure. Furthermore, there is a stationary ring seat and movable disc which controls the fluid flow. This disc is lowered and raised by turning the handle. The flow rate is maximum when the disc is completely raised. On the other hand, the flow is shut off when the disc is lowered completely.

There are three designs of the globe valve known as:

- Z-body type valve

- Y-body type valve

- Angle valve

USES OF GLOBE VALVE

- Globe valve is used in applications where variable pressure is required in the pipeline such as emergency bilge suction line etc.

- It can also be used as a non-return valve. The type of valve where the disc is not attached to the spindle is called screw down non-return valve.  This type of valves is used for preventing back flooding.

- It is mostly used in applications where pressure drop is required.

PRESSURE RELIEF VALVE

The pressure relief valve is a safety valve that works to protect a system or equipment failure during an overpressure event. It is designed to open at a predetermined pressure and relieve the pressure by allowing the pressurized fluid to exit from the system. The valve automatically closes once the system pressure reaches a safe level. The pressure relief valve’s basic purpose is to protect human life and property by venting the fluid out of over pressurized pipes and vessels. Moreover, these valves are also used to maintain sustained pressures downstream or upstream from the valve.

USES OF PRESSURE RELIEF VALVE

- The pressure relief valve is important in applications where pressure levels are crucial for fine operations such as steam, air, or gas power generation.

PINCH VALVE

Pinch Valves, also known as clamp valves, come with a flexible tube that can be pinched mechanically to open and close the fluid path. It offers a straight flow path to fluids without any moving parts. Majorly, it has three parts, a housing, an elastomeric tube, and end connections. Plus, there is a pinch mechanism to pinch the tube and create full closure of flow. The pinch mechanism can be a screw-operated mechanism with a pinch on either one side or both sides. Also, the air or hydraulic pressure mechanism can be used to squeeze the tube. The pinch valve is categorized into two forms known as:

- Open pinch valve

- Enclosed pinch valve

USES OF PINCH VALVE

- Pinch valve is ideal for granular products, slurries, and foodstuffs.

- It is mostly used in places with low temperature due to the elastomeric tube.

PISTON VALVE

The Piston valve uses a piston to regulate the flow of fluid along a tube or pipe in a chamber. This valve can completely open and close the flow path; it is not designed for throttling applications. When opened, only the bottom face of the piston contacts with fluid while the rest of its body remains protected by the upper sealing rings. Hence, sealing rings are saved from erosion by fluid. It is mostly operated manually, but other actuation methods like electric and hydraulic can be used. There are two kinds of piston valves,

- Balanced

- Unbalanced piston valves.

USES OF PISTON VALVE

- A balanced piston valve is ideal for high-pressure services,

- Unbalanced piston valve works in low-pressure systems.

- A piston valve is used for handling the flow of gases and hazardous liquids.

BALL VALVE

Ball valves are suitable to start and stop the fluid flow using a ball-shaped disc located in the valve body. The ball has a port at the center, which comes in line with the direction of flow when the valve is open. While during closed position, the port goes perpendicular to the valve end and blocks the flow path. The valve body is constructed from plastic, ceramic, or metal, while the seat is made of nylon, neoprene, etc. It has a long working life and requires very little maintenance. However, it must not be used for slurries because suspended particles can accumulate and damage the valve.

USES OF BALL VALVES

- Ball valves are used for quick on/off operation in ships.

- Moreover, this valve is associated with low-pressure drop and low leakage.

- These valves have services in many industries, including high-temperature and high-pressure applications.

DIAPHRAGM VALVE

The diaphragm valve is a bi-directional start/shut valve that manages fluid flow by regulating the area from where the fluid enters and exits. It uses a flexible disc, diaphragm that sits on the seat at the top of the valve body to seal the fluid’s passage. The diaphragm is an elastomeric and pressure-responsive membrane that opens, closes, and regulates the valve. Moreover, the valve body can be made of different materials depending on the intended use. Generally, it is made of plastic, metal, or even wood. The diaphragm valve is similar to a pinch valve, but it employs a linear compression that pushes the diaphragm towards the valve body. It can be actuated both manually and automatically to get a fine control overflow.

USES OF DIAPHRAGM VALVE

- It is ideal for moderate pressure and temperature applications.

BILGE INJECTION VALVE

Bilge injection is a valve that enables the engine room bilges to be pumped out directly overboard in the event of an emergency such as flooding. The valve is normally fitted to the end of a branch connection with the main sea water suction line. This enables large main seawater cooling pumps to be used as a bilge pump in an emergency

USES OF BILGE INJECTION VALVE

- The bilge injection valve is used to prevent flooding of the ship. It is a direct suction from the machinery space bilge which is connected to the largest capacity pump.